2” x 6” Pressure Treated balcony/deck installation

The intent of this project is to rebuild/replace the specified decks from a closed surface deck to an open surface deck.  All new material shall be Pressure Treated Southern Pine or Douglas Fir, #2 or better.  New flashing shall be installed at the perimeter of the building and saddle flashing shall be installed at all of the beam pockets. All existing joist shall be cut flush with the wall and a new ledger board installed.  New 4” x 4” post shall be installed to support the deck framing.  All connections shall be made with the appropriate Simpson ® hanger or post connector.

1.	The Contractor shall remove all decking, structural members and handrails from balcony and dispose of offsite to a legal dumpsite.

2.	The Contractor shall remove the siding and examine all structural framing for hidden rot damage and replace as required using #2 or better, treated lumber of like dimension. The Contractor shall notify the Project Manager in advance of any work required that is beyond the original scope of work intent.

3.   	The Contractor shall provide and install 3” x 3” L – metal flashing between the siding and ledger board and install a saddle flashing at all beam pockets into the siding.  The Contractor shall provide and install 15# felt paper assuring that it laps properly between the existing building paper and the new flashing.  If no building paper is found to exist then the top of the flashing shall be sealed to the sheathing with a self-sealing bituthene strip approximately 6” in width.

4.      The Contractor shall provide and install one new wooden patio decks complete with handrails. The deck is to have a 1/8" per foot slope away from the main structure.  All framing material shall be sized per the Southern Pine Council Design Values publication.  All joists shall have a minimum size of 2” x 8” with a maximum spacing of 24” on center and all deck boards shall be 2” x 6”.  All nails, screws, fasteners, and metal are to be hot dipped galvanized or electroplated zinc of the type, size and number required for the job.

5.  	The Contractor shall install a new handrail to match the existing handrails configuration but with a minimum height of 42” and a maximum opening with at any point of 4”.  All handrails shall be constructed to meet all current code requirements. The handrails shall be painted to match the existing.

6.	The Contractor shall caulk all joints with Vulkem 116 polyurethane caulking prior to painting.