# Cedar Fence installation

1.	The Contractor shall install all posts at 8’ on center.  The holes shall be dug no less than 30” below grade in each location.  Eight (8) inches of gravel will be install on the bottom of the hole for drainage. All post shall be made of 4" x 4" x 8' Pressure Treated Southern Pine or Douglas Fir, #2 or better.  All posts shall be set in eighty pounds (80 lbs.) concrete no less than twenty four (24") inches in depth and be allowed to completely set up before any panel attachments take place.

2.	The Contractor shall install fence pickets constructed of (1"x 6"x 6' merchant grade or better dog eared) or (1”x 6”x 6’ merchant grade cut top) cedar securely fastened in place with 2” zinc coated screws.

3.       	The rails shall be 2"x 4" pressure treated, spaced 32” on center with no spans greater than eight  (8') feet in length, fastened to the posts with Simpson ® Wood Fence hangers and hot dip galvanized nails.

4.	The fence shall not exceed 6 feet in height above finished grade.  The top line of the fence shall be held level or level sloping with the grade as specified by the Project Manager

5.	The bottom of the pickets shall be a minimum of one (1") above finished grade in order to prevent any ground contact.  Trim pickets as required.

6.	The pickets shall be spaced a maximum of a quarter inch (1/4") apart form each other.

7.	The Contractor shall backfill all postholes leaving a finish grade level with the surrounding grade.