# Chain link fence installation

The Contractor shall supply all labor, materials, tools, and equipment necessary to install a new chain link fence at the location(s) specified.

1.	The Contractor shall notify all local utilities that may have underground lines in any of the areas to receive the new fencing. Any cut or damaged utilities shall be the responsibility of the Contractor.

2.	The Contractor shall install the fence per the following guidelines:

- Place the post holes at ten foot (10') on center, maximum spacing

- The posts shall be 1 7/8” diameter, 11 gage line post and 2 3/8” diameter, 11 gage terminal post set with eighty (80) pounds of concrete per post at a depth of not less than Twenty Four inches (24") below grade in a minimum 8” diameter hole.

- The finished posts shall have all excess concrete removed and disposed off and shall be backfilled level with the adjoining finished grade or leveled off with the adjacent concrete.

- The chain link fencing shall be constructed using 9 gauge hot dipped galvanized wire.  The fencing shall be laced to the rails and tension wire using 9 gage aluminum wire at 6” on center.

- All six foot fencing shall use 1 5/8” diameter, 14 gage top rails fastened to the posts with hot dip galvanized or aluminum connectors.  The bottom tension wire shall be 7 gage steel.