Scope of Service

Pool and Spa Inspection, Cleaning, and Water Treatment

This scope of service includes a minimum of 2 inspections and cleaning of apartment swimming pool(s) and spa(s) per week. They are to include water chemical analysis and addition of appropriate treatment chemicals, as necessary, as well as inspection of component equipment, to include, but not be limited to:  all pumps, heaters, filters and skimmers.  All chemicals are to be provided and delivered to the property by, and remain the responsibility of, the Contractor. This service is a supplement to the daily care provided by the Property Staff.

A written report, indicating items inspected, existing conditions, actions taken and recommendations is to be provided to the Property Manager immediately upon completion of the inspection.  Life safety or code compliance issues are to be brought to the immediate attention of the Property Manager.

Inspection Criteria

- Contractor is to provide to the apartments a copy of documentation indicating appropriate municipal and/or state required certification or licensing to provide services indicated in this Scope of Service.

- Inspect pool/spa area and equipment for life safety or code issues, including, but not limited to, condition of deck surface and joints, coping, and coping to deck joints, pool and step surface, handrails, existence and availability of safety equipment and markings including pool rules and time of operations, and correct operation of gates.  Record any concerns or recommendations and report deficiencies immediately to the Property Manager.

- Skim pool surface for removal of floating debris.

- Inspect all skimmers.  Clean baskets and remove any debris.  Inspect intakes for blockage and to insure appropriate intake of pool water.  Remove any obstructions to flow.

- Brush walls and steps only with equipment specifically designed for the purpose and for the particular surface of the pool.

- Vacuum pool with a vacuum head designed for the purpose of removing bottom and side debris.  The vacuum head is to be designed to prevent damage or marring of the pool surface.

- Check pressure at gauge on filter.  If pressure exceeds the starting or clean pressure by 8 psi or more, backwash or clean the filter in the appropriate manner for type of filter installed, per the manufacturer’s specifications.  Bleed trapped air from filter using air-relief valve. Record findings and action taken.

- Clean basket strainer upstream of pump, removing debris and check condition of o-ring seal prior to replacement.  If o-ring is damaged, replace.  Check pump for correct operation.

- All chemicals for pool water treatment are to be supplied by Contractor and carried to and from the property as a part of the process of each inspection.  No chemicals are to be stored on the property by the Contractor.  Chemical containers of any kind are to be appropriately identified, stored and handled.  MSDS sheets are to be provided for all chemicals in use by Contractor.  Correct storage and use of chemicals remains the responsibility of the Contractor.  Record and report immediately to Property Manager any spills or similar incidents that may occur. The Contractor is to use the same type and quality of chemicals as the property.

- Check pool water for total alkalinity.  Proper range is 80-120 parts per million (ppm) for plastered pools, 100-140 ppm for vinyl liner pools.  If below or above, add appropriate chemicals at the proper rate for size of pool and desired increase or decrease.

- Test pool water for pH level.  Proper range is 7.4-7.6.  If above or below, add appropriate chemicals at the proper rate for size of pool and required increase or decrease.  Do not attempt to adjust pH until appropriate alkalinity range has been reached.  Retest the water 4 hours after adjustment and retreat if necessary.

- Test pool water for chlorine residual.  Proper range for residual chlorine is at least 1 ppm.  Appropriate range for free chlorine is 1-3 ppm.  If chlorine level is below range, add appropriate type of chlorine product for specific installation and need.  Adjust regular chlorination as necessary.  Shock or super chlorinate as necessary to remove swimmer waste, bacteria and algae.  Contractor is to be responsible for insuring pool safety during this process, including, but not limited to, closing of pool, insuring lack of access to pool and retesting of water prior to reopening pool.

- Record all actions taken regarding the chemical level of the pool water, including all chemicals used and the method used to introduce them as well as the rate or volume at which they were added into the pool water.

- Inspect any pool or spa heater for correct operation. Have the inlet gas pressure checked and adjusted yearly as necessary, recording pressure and adjustments.  Inspect combustion air intake and insure unobstructed airflow to heater.  Test temperature rise across heat exchanger to insure rise is within manufacturer’s specifications.  Test flow switch and any safety control devices for correct operation.  Record results and recommendations in report.  Life safety or code compliance issues are to be reported to the Property Manager immediately.

- Test swimming pool lighting for proper operation. Report any lamp burnouts to the Property Manager.

- Train and instruct the property staff on intermittent testing of pool water conditions and how to record daily readings. Instruct the property maintenance staff on the appropriate handling and storage of pool chemicals, and to adjust the chemical balance of the water for total alkalinity, proper pH and chlorine residual levels.

- Inspect entire pool system for correct operation and provide Property Manager with written report of total conditions and action taken.

- The vendor shall complete the Essex pool log at each visit. Upon leaving the office the log shall be given to the Manager.

Follow Essex’s procedures of winterization of the pool and equipment. The procedure will be provided as an addendum to this scope of work.

POOL WINTERIZE STEPS

Date

# To close for winter							      To Do	     Completed	       Initial

- Back wash filter  									        _______	        _______

- Drain DE filter tanks & leave backwash valve open.					        _______	        _______

- Open sand filters									        _______	        _______

- Unplug filter drain plug – leave off							        _______	        _______

- Blow out all water from multiport valve.						        _______	        _______

- Disconnect the pump & filter							         _______	        _______

- Remove all water from the pump							         _______	        _______

- Remove all drain plugs								         _______	        _______

- Drain the heater									         _______	        _______

- Remove plugs & blow all water out of lines						         _______	        _______

- Unscrew & disconnect all fittings and/or unions at pump and filter system		         _______	        _______

- Remove all return jet fittings (entire fitting)						         _______	        _______

- Remove all skimmer baskets							         _______	        _______

- Blow out all return jet pipes								         _______	        _______

- Blow out all skimmer lines								         _______	        _______

- Install Gizzmo – type plugs in the skimmer						         _______	        _______

- Blow out all lines to fountains, slides, or other accessories				         _______	        _______

- Blow out main drain line and close the line off in the pump room			         _______	        _______

- Seal off or duct tape all exposed pipes to prevent water from entering them		         _______	        _______

- Remove all ropes, floats, diving boards, and slides					         _______	        _______

- PH level set at 7.2 – 7.6								         _______	        _______

- Alkalinity set at 100-150ppm							         _______	        _______

- Chlorine level above 3.0								         _______ 	        _______

- Drain water level below tile lines							         _______	        _______

- Place cover on the pool								         _______	        _______

Pool Winterizing Explanations

Item 1-5	Backwash the filter very well to clean it out.  Drain DE filter tanks and leave backwash valve open.  On sand filters, unplug the filter drain and leave off.  Put drain plug with other removed items in the pump basket.  Make sure multiport valve has no water in it.  Blow it out with a compressor or shop vacuum if necessary.  Please note that it is not recommended to “acid wash” DE filters at the time of the pool closing.  This is best to do in the spring so that you can immediately run pool water through the system.  It is not good to use muriatic acid on a DE filter and then just rinse it off and put it away.  The acid may degrade the filter parts over the winter.

Item 6-8	Disconnect your pump and filter.  Make sure that water is totally drained from the pump.  Turn pump upside down once to make sure!  Remove any drain plugs from the pump.  It is a good idea to store any small plugs or parts in the pump basket.  This way you will be able to find them easily in the spring.

Item 9-10	If there is a heater, drain it and make sure there is not sitting water inside.  Blow it out with a compressor or shop vac.  Drain heater totally and remove all drain plugs (if any).  Put drain plugs in the pump basket for safekeeping.  We do not recommend removing the heater tray.  You can remove it if you want, but you may have trouble putting back in the spring.  It is not necessary on most units.

Item 11	Unscrew and loosen any quick disconnect fittings or unions at your pump and filter system.  Remember, the name of the game is “no freeze cracks”.  If the water is all drained out of your pipes and fittings, it cannot freeze and expand and cause the equipment to crack.

Item 12-13	Remove all return jet fittings (the entire fitting!).  If you crack a fitting while removing it, don’t panic!  You can get a replacement come spring.  Remove all skimmer baskets.  Put fittings and any other items that you remove in one of the skimmer baskets or the pump basket to avoid loss (this includes the dive board bolts too).

Item 14	Blow out all return jet pipes using an air compressor or shop vac.  Hook up air compressor or shop vac to the return lines at the filter system – or – some people prefer to screw the compressor fitting into the drain plug of the pump.  This will give a good seal and allow you to blow out the entire system from that one spot – but this is up to you.  Keep the air blowing until the air bubbles start to become visible from the return jets in the pool.  Put a plug in the fitting under the water when you see the bubbles blowing at full force.  This will mean that 99% of the water is out of the pipe.  Make sure plug is in tight!  This is most important.

Item 15-17	Blow out all skimmer (suction side) pipes in a similar fashion as noted in #6.  Put a Gizzmo-type screw in plug in the skimmer when bubbles start to become visible.  We know that this is sometimes difficult, but proper gizmo installation is important.  Make sure that you put Teflon tape on the gizmo threads before installing.  This insures a tight seal.  If you do not want to use Gizzmo plugs and want to use black rubber-type plugs instead, that is OK as long as there is something in the skimmer to allow for water expansion when it freezes.  Usually a closed plastic empty soda-type bottle will work.  This is very important!  Do not just plug the skimmer lines and forget about them.  Water can easily freeze in a skimmer and crack the plastic.  This would be bad!  Also, if you have a slide, an auto vac system or a waterfall, you will have to drain and blow out those pipes as well.  Remember – we do not recommend putting anti-freeze type products in the pipes.  You will not need it if the lines are properly blown out.  The anti-freeze can cause a mess in the Spring when you go to start your system and it gets sucked into your filter and blown back into the pool.  Try to avoid antifreeze – if possible - by properly evacuating all the water from the pipes.

Item 18	Blow out main drain line (if any).  No, you don’t have to dive down and plug the drain pipe.  When you see bubbles coming out of the drain, plug the pipe on your end or close the gate valve.  This is as much protection as you can give to a main drain line.  By doing this you will cause an “air lock” in the line and no more water should enter the pipe from the pool side.

Item 19	Put duct tape on all exposed pipes to prevent anything from getting into them.  Use a lot of tape, it’s cheap!

Item 20		Remove rope and floats from pool and put with the rest of the supplies.

Item 20	Remove dive board and ladders.  Put in a safe spot – a shed or the garage.  Put the pump and filter in the shed or garage as well.  You probably are not going to want to move your filter if it is a sand filter.  You can leave that outside!  Remember… do not lose dive bolts or ladder bumpers.  Put them in the skimmer or pump baskets.

Item 21-23	Mix any granular winterizing chemicals in a bucket so that they are totally dissolved.  Dump mixture into the pool.  You want to avoid any undissolved granules from settling on the pool floor and staining the liner.  This is very important.  If you are using any liquid winterizing chemicals, pour them in the pool as well.  Test the pool for PH and Total Alkalinity.  Adjust to normal levels using PH PLUS or MINUS and ALKALINITY PLUS.  PH should be between 7.2-7.6 and Alkalinity between 100-150ppm.  Make sure one of your winterizer chemicals consists of a SHOCK-type product.  You want the chlorine level in the pool to be rather high (over 3.0 for wintertime).