## GUTTER AND DOWNSPOUT INSTALLATION

Install roof gutters and/or downspouts, fabricated from sheet metal, with straps, adhesives and anchors as specified to drain roof in the most efficient manner.

### PRODUCTS AND MATERIALS

Gutters and/or downspouts shall be fabricated and installed using the following:

1.	Gutter - Prefinished aluminum or galvanized steel, 5” trough

- Downspouts - Prefinished aluminum or galvanized steel, 3” X 4” minimum

3.	Splash Blocks - Precast concrete, standard size

4.	Fasteners - Concealed gutter brackets @ 24” o.c.

5.	Downspout brackets - spaced 8’-0” o.c.

### FABRICATION AND INSTALLATION

      - Before soldering, clean surfaces to remove oils and foreign matter.  Pretin edges of sheets to be soldered to a width of 1 ½”, except where pretinned surface will show in finished work.  Do not use torches for soldering.  Heat surfaces to receive solder and flow solder into joint, filling completely.  Remove flux and spatter from exposed surfaces.

2.	Form nonexpansion (but movable) joints in metal to accommodate elastomeric sealant to comply with SMACNA standards.  Fill joint with sealant and form metal to conceal surfaces.  Use joint adhesive for nonmoving joints that are not soldered.

3.	Fabricate nonmoving seams in aluminum with flat-lock seams and seal with epoxy seam sealer.  Rivet joints for additional strength.

4.	Separate metal from noncompatible metal or corrosive substrates by coating concealed surfaces with asphaltic mastic.

5.	Contractor shall design the roof drainage system for the slope of the roof, total area to be drained, capacity of gutters and downspouts and location of downspout outlet.

6.	No downspout shall discharge directly onto another roof or enclosed patio without approval of the Project Manager.

7.   Gutter style shall match existing or be OGEE if not specified.

- Maximum spacing of downspouts shall be 25’-0” o.c.

- The front edge of the gutter shall be lower than the back to allow overflow away from roof.

- Gutters shall be supported by hidden straphangers with sufficient strength to support the system when full of water or ice.

- Straps shall be fastened to the roof rafters or wall studs with minimum #12 - 3” wood screws at 24” o.c. or screw shank nails.

- Form and install a downspout funnel where two downspouts joint together.  Use an oversize downspout if applicable.

- Solder or seal joints with an aluminum-bodied sealant manufactured to seal metal assemblies.

- Slope gutters 1” for every 40 feet of run.  Adjust for variations in the roof plane.