INTERIOR PAINTING

Bids should be submitted on a per apartment basis.  Pricing should be submitted on:

One bedroom, one bath	$_____________

Two bedroom, one bath	$_____________

Two bedroom, two bath	$_____________

Three bedroom, two bath	$_____________

Contractor shall supply labor to complete the following work in each unit.  Paint will be supplied by the property.

SCOPE OF WORK

- Remove outlet covers, switch plates, wall plates and escutcheons and place in a kitchen drawer.

- Ceiling fixtures, phone jacks, thermostats and other non-moveable items should be masked.

- Caulk any gaps along trim base or casing.

- Contractor shall remove any black vinyl cove base prior to painting.

- Prep walls.  Fill nail holes, nail pops, stress cracks and drywall damage less than four inches in diameter.  Scrape or sand imperfections protruding from the wall or ceiling.

- Prime patches to prevent “flashing” of the final coat.

- Drop cloths must be used throughout the apartment during painting.  Tarp all appliances and cabinets.  Contractor is responsible for all paint clean up.

- Tape or otherwise cover fire sprinkler heads to prevent paint over-spray from coming in to contact with the head.  Contractor is responsible for notifying the Property Management of any existing over-spray prior to commencement of the painting project.  Contractor will be held responsible for any over-spray found on the fire sprinkler heads at completion of the project.

- Property Manager shall be notified in writing of any water infiltration, water stains and mold or mildew prior to commencement of painting project.

- Clean or seal smoke, grease or other films as required to assure proper adhesion of new paint.

- Remove over-spray from windows, light fixtures, cabinets, door hardware or other surfaces not specifically scheduled for painting at completion of project.  Clean unit of debris related to project and remove supplies, equipment and trash prior to scheduling the final inspection with the Property Manager.

- The following  areas shall be painted in semi-gloss latex enamel:

- Kitchens and bathrooms

- Closet doors on both sides in hallway areas, apartment doors and frames, closet shelving, baseboards and windowsills if indicated on work order or if already painted.

The remainder of the unit shall be painted in (flat latex, low luster latex, or eggshell finish) as directed by management.

Any additional work in the apartment must be approved in writing by management before commencement.

Provide a unit price for each additional item as noted in the proposal:

A)___________________________		$_________ each

B)___________________________		$_________ each

C)___________________________		$_________ each