Railroad tie retaining wall replacement

The scope of work includes removal and replacement of railroad tie retaining walls and installation of  guard rail fencing.

1.  	Remove existing walls as shown on the attached site plan and dispose of

all debris in containers provided by the contractor.  All debris shall be

removed to a designated storage area on a daily basis.

2.  	Regrade area as required for installation of new wall.

3.  	Construct new retaining walls with #1 railroad ties.  Deadmen shall be

spaced at 8’– 8” on center, every other row.  Deadmen length shall be

equal to the height of the wall or 4’-0”, minimum.  All walls require at least

one row of deadmen.

4.  	Each end of the railroad tie shall be fastened with a minimum of 4-60d

common nails, two side nailed and two end nailed.

5.  	The back face of all walls shall be lined with a continuous layer of filter

cloth.

6.  	A minimum, one foot wide by 18” high layer of gravel shall be installed

behind all new walls.  All soils shall be compacted during the construction

of the wall to prevent future settlement of the soil.

7.  	Notify onsite management of any damaged sprinkler lines or utilities to

allow for repair prior to backfilling.  The contractor is responsible for

coordinating all repairs.

8.  	Backfill and fine grade all disturbed or damaged areas with topsoil and

sod.

9.  	All retaining walls over 48” in height require a 48” high guard rail fence.

The guard rail fence can be constructed with 4”x4” Pressure treated

Southern Pine or Douglas Fir posts at 8’-0” on center with two (2)

horizontal rails spaced at 24” on center or as specified by the Project

Manager.