## Tree trimming requirements

Pruning shall be conducted as follows:

The pruning required for this project shall be considered a Class 1 prune which is defined as “ Major trimming from the main trunk to the tip of the branches and thinning of branches or cross branches as required to meet the following criteria.

The contractor shall notify the Project Manager 7 days prior to starting the removal or pruning of trees on the site.  A schedule shall be prepared indicating the day of the month and the building on the site where the contractors personnel will be working on and this schedule shall be updated as it changes.

The contractor shall protect all existing plant material which is to remain, utilities, paving and other facilities from damage caused by the landscaping operations.

- Remove all broken  or dead branches.

- All cuts shall be made with sharp instruments and shall be flush cut with trunk or adjacent branch to insure elimination of stubs.  “Headback” cuts at right angles to line of growth shall not be permitted.

- All trimmings shall be removed from the site on a daily basis.

- At least one paved pedestrian access route to the building shall be kept clean at all times.

- All pruning shall be by methods using Crown thinning, Crown raising or Crown reduction.  The Project Manager shall be notified in advance of any trees which the Contractor plans to prune by the Tipping or Topping method.

- The lowest branch on all trees shall be pruned to provide a minimum clear height of 8 feet to the surface grade as measured from the lowest point on a branch.

- All trees shall  be trimmed to provide a minimum clear distance of 6 feet to all structures.

- The contractor shall make all cuts in a manner that prevents tearing of the stem tissue or bark as much as is feasible.