- Instruction to Bidders

Bidders are required to inspect each property and assess the specific field conditions and prepare a unit price for the individual items as noted in the attached Bid Proposal Form.   No adjustments will be made or considered for field conditions or materials not included in the Bidder’s proposal if the:

a.	materials or work in question could have been reasonably determined or included based on industry standards and accepted practices;

b.	materials or work in question could have been reasonably determined or included based on a visual inspection of the work site;

- work or materials in question were listed or reasonably inferred in the bidding documents;

NOTE: The preferred paint material supplier for this project are:

ICI Dulux Paint

Or

Sherwin Williams as may be noted by the Project Manager

Bidders are advised to contact the paint material supplier regarding specific type of paint product for this work prior to submission of their proposals.

Working hours shall be considered to be as follows:

Monday through Friday		8:00a.m. until dark, or 6:00PM, which ever is earlier

Saturday:			9:00 AM to 5:00 PM

Sunday:			No work on Sundays.

Holidays:			No work shall be performed on legal holidays.

Contractor shall have a designated on site Project Manager with good written and verbal communication skills for each project.  The Project Manager shall be at the property every working day for the entire duration of the project.  After commencing work each day, the Project Manager, shall contact the manager in order to discuss problems incurred during the previous day’s work.

- Contractor is responsible for all housing costs of the Contractor or sub-contractors crews.  Contractor shall specify in their proposal if housing is required and the number of units needed to house their crews. Essex will only provide assistance to the contractor for securing housing and does not guarantee that housing will be available at the property where the work is being performed.

- No Contingency Costs should be included in proposals.  All adjustments shall be done by change order.

- All bids must be submitted on the attached bid form, and must include unit pricing.  Attached also is a sample of the building Talley sheet that will be required to be filled out per field conditions and before contract execution. Please note that each building Talley sheet will be specific for each property. Building Talley Sheets shall be used to establish quantities for replacement material, as well as used for basis of verification of replacement. Supplemental information that the bidder may wish to transmit to the Owner may be furnished as an attachment to the bid form.

- Format of the Contract format for this project is available upon request from the Project Manager.  Submission of a proposal for this project shall indicate the bidder’s acceptance of those terms and conditions unless objections to those terms and conditions are stipulated with the proposal.

Exhibit “A” - Work Scope

SCOPE OF WORK:

The intent of these specifications is for complete exterior renovation and repair on all buildings including the Leasing Office/Clubhouse, Mail Kiosks, pool and property fencing, building and directional signs, and other structures previously painted and present at the referenced communities.

The scope of work shall be site specific, and it may include but not be limited to

- Painting of all previously painted surfaces including, all non maintenance free siding (exclude maintenance free siding such as vinyl, brick, aluminum siding), Chimney caps, wood trim and rails and the Pool House, and mail kiosks, property and pool fencing, building signs, and other previously painted structures that present at the community.

- Repair and/or Reconstruction of the balconies.

- Replacement of the rotted trim, fascia, balcony hand rails, balusters, joists and trim board.

- All replacement wood trim material shall be six sided primed.

- Replacement or securing of missing, damaged or loose aluminium/vinyl including all accessories associated with aluminium/vinyl siding or metal fascia wraps.

- Re-flashing and repair of the brick rowlock if present.

- All replaced trim and siding must be properly flashed.

- Repair and or replacement as necessary of any deteriorated siding.  This includes replacement or re-nailing

- Repair and painting of the siding and ceilings in the breezeways.

- Repair of breezeway stairs

- Chimney caps, Gutters and downspout repairs and replacement on all buildings

- Cleaning and aligning and re-securing of the gutters throughout the property.

- Painting of roof metals.

12. 	       Painting of all previously painted surfaces unless specifically called out.

Special Notes:

Where EIFS is present, Contractor/Sub-contractor for the EIFS repairs must be a certified installer of EIFS products.  A copy of the contractor’s certifications must be provided at the time of Bid submission.

# BALCONY REPAIRS.

- Inspect all balconies, and replace all deteriorated railings, joists, trim, railings and decking

- Inspect all columns on all balconies and repair/replace as necessary

# REPLACEMENT OF ROTTED TRIM, FASCIA, HANDRAILS, JOISTS AND TRIM BOARD

- Contractor has observed the field conditions and performed a survey of the property for the field conditions present that would affect unit pricing.  Exact quantities will been determined by onsite inspection prior to contract signing and will be included in the building Talley sheets to be attached as part of the contract to complete this project.

- As part of the proposal the Contractor shall furnished unit pricing for any hidden damage not visible to a naked eye. Unit pricing includes, material and labor with all accessories as required for a complete job.  The unit pricing shall be used for any hidden damages encountered as part of this project.  No repairs are to proceed without the written authorization of the Project Manager, and a signed Change Order.

- All new materials are to be free from defects and are to match existing, except where otherwise noted.  All new materials are to be certifiable as being "asbestos free" by the manufacturer.

- Replace all damaged, dry rot deteriorated or delaminating wood and trim on buildings including all storage/utility room doors and trash enclosures and fencing throughout the property.

- Proper flashing must be used for all replaced material.  All flashing must be formed on site and installed in accordance with standard practices.  Special care should be exercised on placement of flashing at the corners to insure that proper seal is achieved both horizontally and vertically.

- If decks or balconies, or any of their structural components, require replacement, no modification of the structural framing is to be made.  All deck replacements shall follow the existing layout.

- All work is to comply with current building codes.

- New Railings and baluster spacing are to comply with current building codes and not constructed to match existing, where the existing spacing does not meet code.  This includes railing height, baluster spacing, riser height and tread sizing. If modification to railings is required, prior to installation of new railings, the issue shall be discussed with the Project Manager and a written approval for the modification shall be issued.

- Where buried or hidden damage is encountered, which exceeds materials takeoff indicated in specification and scope of work, Project Manager is to be notified for a written approval to proceed.  However, except where the structural integrity of the building has been compromised, the scope of work is to include "sistering” to sound members as necessary to provide nailable surfaces.  Rotted and deteriorated materials are to be removed as necessary. Any sign of termite or insect infestation or structural damage is to be reported to the Project Manager immediately.

- Only pressure treated materials are to be used for ground/concrete surfaces or plates.  No new and replaced wood shall be in direct contract with a masonry/concrete surface. In such cases the wood must be separated from the concrete/masonry with an approved moisture barrier. All miters and laps are to be tight, and properly formed.  Contractor is to replace any materials exposed to weather that are not properly lapped or mitered.

- All deck/balcony/walkway railings are to be inspected as part of this project and reset or re-secured or replaced as necessary.  Railing hangers are to be securely fastened to solid wood members or concealed blocking.

- It is the intent of this project to repair or replace, as is required by field conditions, damaged and unsound materials.  Materials to be replaced shall be replaced with similar materials.  Similar shall mean that the item in question shall have the same composition, same look, same life span, same use and same cost.  Contractor is responsible for maintaining the continuity of appearance on the buildings between existing siding, trim, windows, fixtures, etc. and those materials being used to fulfill the contract scope of work.  The Owner shall retain the right to approve all materials used in the course of this work.  Contractor also agrees that what may be an appropriate use of a material on one building may not constitute an acceptable use on a different building.

REPAIR OF THE BRICK ROWLOCKS WHERE APPLICABLE.

- Place tarps, drop clothes, etc., as needed to protect surroundings.  Particular attention should be given to windows, balconies, and patios.

- Ladders may be left on the property during the time that work is being done, however, they must be secured at night to eliminate use by unauthorized persons.  Owner accepts no responsibility for damage or loss of contractor’s equipment.

- Contractor shall be responsible for repairing any damaged landscaping, roofing, etc.

- Remove existing caulk from in between the siding and the brick rowlock, where the rowlock has failed.

- Remove all loose or defective mortar and brick, rake and brush to a solid bed joint.

- Grind out mortar 1/2” or to a solid bed joint in the areas of repair

- Remove all dust & loose particles from the ground joints.

- Flush with water to remove all debris and to dampen surface to prevent masonry from pulling all of the water from the mortar.

- Tuck point solid mortar joint in a closely matching mortar color.  Joints should then be pointed and tooled concave to match the existing mortar.

- Replace all existing failed rowlock.

- All new brick shall match existing.  Samples of brick shall be furnished to the Project Manager for approval prior to installation.

- All Brick work shall comply with Section 04210.

RE-FLASHING OF THE BRICK ROWLOCKS WHERE APPLICABLE.

A.	Place tarps, drop clothes, etc., as needed to protect surroundings.  Particular attention should be given to windows, balconies, and patios.

- Ladders may be left on the property during the time that work is being done, however, they must be secured at night to eliminate use by unauthorized persons.  Owner accepts no responsibility for damage or loss of contractors equipment.

- Contractor shall be responsible for repairing any damaged landscaping, roofing, etc.

- Remove existing caulk from in between the siding and the brick rowlock.

- Remove all loose or defective mortar and brick, rake and brush to a solid bed joint.

- Grind out mortar 1/2” or to a solid bed joint in the areas of repair

- Remove all dust & loose particles from the ground joints.

- Flush with water to remove all debris and to dampen surface to prevent masonry from pulling all of the water from the mortar.

- Tuckpoint solid mortar joint in a closely matching mortar color.  Joints should then be pointed and tooled concave to match the existing mortar.

- Replace all existing failed rowlock.

- All new brick shall match existing.  Samples of brick shall be furnished to the Project Manager for approval prior to installation.

- All Brick work shall comply with Section 04210.

- Install a bead of caulk on the brick prior to the installation of the new flashing

- Extend the new flashing a minimum of 4” under the siding.

- Flashing to be 24 gage galvanized with Kaynar finish.  Color samples to be submitted to the Owner for final color selection.

- Flashing to extend over the entire horizontal surface of the brick and extend down the face of the brick a minimum of ¾”, with a “drip edge” edge treatment to direct the water away form the face of the brick.

- All seams to be overlapped a minimum of 6”

- All seams to be caulked with Solar Seal rubberized caulk or equivalent

- Reinstall boards and paint as necessary

- Caulk all nail heads used to repair of siding.

- Repair the siding per specifications noted elsewhere in these specifications.

EIFS REPAIR WHERE APPLICABLE:

PROJECT CONDITIONS

Do not apply coating materials to exterior surfaces until ambient temperature, and surface temperature of surfaces to receive coating materials, is above 40 degrees F (4 degrees C) and is expected to remain so for 24 hours after application of finish coating.

SEQUENCING

NOTE ** Retain paragraphs that apply to project.

A.	Ensure that sheet metal flashings and trim adjacent to EIFS system repairs are repaired and in good condition after the repair is made.

- Ensure that joint sealers adjacent to EIFS system repair are repaired immediately after finish coating has dried.

MANUFACTURERS

B.	Acceptable Manufacturer:  Sto Finish Systems Division of Sto Corporation; P.O. Box 44609, Atlanta, GA 30336-5609; ASD. Tel: (800) 221-2397 or (404) 346-3666, Fax: (404) 346-3119.

NOTE ** Delete one of the two following paragraphs; coordinate with Division 1 requirements.

C.	Substitutions:  Not permitted.

D.	Unless otherwise specified for an individual product or material, supply all products specified in this section from the same manufacturer.

JOINT SEALANT:

- Joint Sealant:  Comply with EIMA Standard 300.01; minimum 50 percent elongation.

MIXES

E.	Mix materials specified to be site-mixed in accordance with manufacturer's mixing instructions; do not dilute finish coatings except as instructed.

F.	Mix only enough materials that can be applied during working time recommended by manufacturer; do not re-temper mixes.

G.	Do not add anti-freeze admixtures or other admixtures that may adversely affect performance or appearance of coating materials.

EXAMINATION

NOTE ** The EIFS manufacturer is not responsible for design of the substrate, including its structural support.  The following are some design considerations for substrates:

NOTE **

NOTE ** Design for maximum allowable deflection, normal to wall plane, of L/240 for PB System, L/360 for PM System; include wind load requirements.

NOTE **

NOTE ** Prevent accumulation of water behind coating, by condensation within substrate or by water leakage.

NOTE ** Delete requirements in the following paragraph not applicable to project.

H.	Prior to installation of the repair patch, inspect substrates to receive EIFS systems for the following:

1.	Contamination:  Presence of foreign matter which might affect coating performance or appearance.

2.	Delamination, damage, defects, or deterioration.

3.	Cracking:  Measure widths of cracks; record locations.

4.	Surface absorption and chalking.

5.	Moisture content:  Use moisture meter of type recommended by manufacturer to determine if moisture content is within limits recommended by manufacturer.

6.	Surface plane within tolerances required in manufacturer's instructions.

NOTE ** Delete substrates that are not required.

7.	Installation sheathing conforming to sheathing manufacturer's instructions.

NOTE ** Provide minimum 3/4-inch (19 mm) wide expansion joints in EIFS where they exist in substrate or supporting construction, where EIFS abuts dissimilar construction or materials, at changes in building height, and at floorlines of multi-story wood construction; provide minimum 1/2-inch (13 mm) wide sealant joints at penetrations through EIFS system.

NOTE **

NOTE ** Do not allow coatings to be applied over joints.

I.	Verify that joints in substrates are in accordance with EIFSS manufacturer's recommendations.

PREPARATION

J.	Correct unacceptable conditions before proceeding.

K.	Protect surfaces adjacent to locations of EIFS system installation; do not allow EIFS system materials on surfaces not indicated to receive them.

L.	Prepare substrates to receive EIFS system materials in accordance with manufacturer's instructions and as follows:

NOTE ** Delete any of the following sub-paragraphs not applicable to project.

1.	Remove loose, damaged, or deteriorated materials.

2.	Remove surface contaminants on concrete surfaces in accordance with ASTM D 4258.

3.	Remove surface contaminants on concrete masonry surfaces in accordance with ASTM D 4261.

4.	Repair surface defects using patching and leveling material.

5.	Repair surface cracks using patching and leveling material, or patching and leveling material with fabric reinforcement, according to manufacturer's recommendations for crack size.

6.	Replace damaged sheathing.

7.	Apply surface conditioner to chalked or absorbent surfaces.

8.	Apply skim coat to level substrates to required plane tolerances.

INSTALLATION

M.	Install EIFS system components in accordance with manufacturer's instructions and recommendations.

N.	Apply prime and finish coats in accordance with manufacturer's instructions; do not exceed manufacturer's recommended spread rate for coating materials.

O.	Do not apply coatings over sealant joints, cold joints, expansion joints, or control joints.

P.	Finish final coats to match accepted mock-ups.

Q.	Seal all joints between EIFS system and adjacent materials using specified sealant.

PROTECTION

R.	Protect installed EIFS system from damage by subsequent construction activities.

S.	Repair installed EIFS system, if damaged by subsequent construction activities, in accordance with manufacturer's recommendations, to Owners satisfaction, if damaged areas of EIFS system cannot be repaired to Owner’s’ acceptance, replace system components to nearest adjacent joints.

NOTE ** The following sample schedule may assist in establishing a schedule to identify locations of products in the project; coordinate with drawings.  Delete entire Article if not required.

SPECIFICATIONS

- Where there is evidence of creeping in the lamina, cut the lamina and inspect the sheathing for soundness and signs of moisture penetration and deterioration.

- Replace damaged sections of the sheathing.

- To patch the lamina, grind existing EFIS lamina down to basecoat in 3in. wide strips around the opening.

- Replace EFIS foam insulation (adhere to sheathing), fill gaps between new and old foam board and rasp transition smooth.

- Apply one coat of new basecoat over patch area, and install new mesh (to match existing mesh) over patch area.  Lap new mesh 3 in. over the existing ground lamina (exposed basecoat) Fully embed mesh in basecoat.

- Allow basecoat to cure.

- Install finish coat.  Trowel and apply to meet and match existing trowel patterns and texture of surrounding EFIS.

- The following detail shall be used for the re-caulking of the all expansion joints

# REPAIR/REPLACEMENT OF THE SIDING

- Contractor has observed the field conditions and performed a survey of the property for the field conditions present that would affect unit pricing.  Exact quantities will be determined by onsite inspection prior to contract signing and will be included in the building tally sheets to be attached as part of the contract to complete this project

- As part of the proposal the Contractor has furnished unit pricing for any hidden damage not visible to a naked eye. Unit pricing includes, material and labor with all accessories as required for a complete job.  These unit prices shall be used for any hidden damages encountered as part of this project.  No repairs are to be without the prior authorization of the Project Manager.

- All new materials are to be free from defects and are to match existing, except where otherwise noted.  All new materials are to be certifiable as being "asbestos free" by the manufacturer.

- All work is to comply with current building codes.

- Where buried or hidden damage is encountered, which exceeds materials takeoff indicated in specification and scope of work, Project Manager is to be notified for approval to proceed.  However, except where the structural integrity of the building has been compromised, the scope of work is to include "sistering” to sound members as necessary to provide nailable surfaces.  Rotted and deteriorated materials are to be removed as necessary. Any sign of termite or insect infestation or structural damage is to be reported to the Construction Manager immediately.

- Only pressure treated materials are to be used for ground/concrete surfaces or plates.  No new and replaced wood shall be in direct contract with a masonry/concrete surface. In such cases the wood must be separated from the concrete/masonry with an approved moisture barrier. All miters and laps are to be tight, and properly formed.  Contractor is to replace any materials exposed to weather that are not properly lapped or mitered.

- It is the intent of this project to repair or replace, as is required by field conditions, damaged and unsound materials.  Materials to be replaced shall be replaced with similar materials.  Similar shall mean that the item in question shall have the same composition, same look, same life span, same use and cost.  Contractor is responsible for maintaining the continuity of appearance on the buildings between existing siding, trim, windows, fixtures, etc. and those materials being used to fulfill the contract scope of work.  The Owner shall retain the right to approve all materials used in the course of this work.  Contractor also agrees that what may be an appropriate use of a material on one building may not constitute an acceptable use on a different building.

REPAIR OF STAIRS

- Contractor has observed the field conditions and performed a survey of the property for the field conditions present that would affect unit pricing.  Exact quantities will be determined by onsite inspection prior to contract signing and will be included in the building tally sheets to be attached as part of the contract to complete this project.

- All new materials are to be free from defects and are to match existing, except where otherwise noted.  All new materials are to be certifiable as being "asbestos free" by the manufacturer.

- No modification of the structural framing is to be made if any structural components require replacement,.  All stair replacements shall follow the existing layout.

- All work is to comply with current building codes.

- New Railings and baluster spacing are to comply with current building codes and not constructed to match existing, where the existing spacing does not meet code.  This includes railing height, baluster spacing, riser height and tread sizing. If any railing modifications are required, prior to installation of new railings, the issue shall be discussed with the Project Manager and a written approval for the modification shall be issued.

- Where buried or hidden damage is encountered, which exceeds materials takeoff indicated in specification and scope of work, Project Manager is to be notified for prior approval before proceeding.

- All stair railings are to be inspected as part of this project and reset or re-secured or replaced as necessary.

- It is the intent of this project to repair or replace, as is required by field conditions, damaged and unsound materials.  Materials to be replaced shall be replaced with similar materials.  Similar shall mean that the item in question shall have the same composition, same look, same life span, same use, and cost.  Contractor is responsible for maintaining the continuity of appearance on the buildings between existing siding, trim, windows, fixtures, etc. and those materials being used to fulfill the contract scope of work.  The Owner shall retain the right to approve all materials used in the course of this work.  Contractor also agrees that what may be an appropriate use of a material on one building may not constitute an acceptable use on a different building.

CHIMNEY CAPS, GUTTERS AND DOWNSPOUTS REPAIR AND REPLACEMENT ON ALL BUILDINGS.

- Inspect all Chimney caps, gutters and downspouts for dips, rusts and check seams.

- Contractor has observed the field conditions and performed a survey and takeoff of the property and the exact quantities have been determined by onsite inspection of the field conditions and included in the building tally sheets submitted as part of the proposal to complete this project.

- All new materials are to be free from defects and are to match existing, except where otherwise noted.  All new materials are to be certifiable as being "asbestos free" by the manufacturer.

- No modification of the structural framing is to be made if any structural components require replacement.

- All work is to comply with current building codes.

- Where buried or hidden damage is encountered, which exceeds materials takeoff indicated in specification and scope of work, Project Manager is to be notified for approval prior to work proceeding.

- It is the intent of this project to repair or replace, as is required by field conditions, damaged and unsound materials.  Materials to be replaced shall be replaced with similar materials.  Similar shall mean that the item in question shall have the same composition, same look, same life span, same use, and cost.  Contractor is responsible for maintaining the continuity of appearance on the buildings between existing siding, trim, windows, fixtures, etc. and those materials being used to fulfill the contract scope of work.  The Owner shall retain the right to approve all materials used in the course of this work.  Contractor also agrees that what may be an appropriate use of a material on one building may not constitute an acceptable use on a different building.

- Any Chimney cap requiring replacement shall be replaced.  All replacement chimney caps should have a galvanized “Kaynar” finish over the flood part.  Support new cap as needed with additional 2x4 cross members. Install “X” breaks in the cap for additional support and verify top of the cap has a positive pitch from the middle to all sides for proper drainage. Keep all vent caps of good condition and re-install.  Replacement chimney caps shall extend over the brick on the sides a minimum of 6”.  Galvanized screws shall be used to secure the caps to the brick.  Screws shall be placed at a maximum separation distance of 8” o.c.  Screws shall be sized to securely fasten the caps to the chimneys to withstand winds and weather conditions customary to the region.

- Where gutter or downspouts require replacement the following products shall be used:

  - Gutter - Prefinished aluminum

  - Downspouts - Prefinished aluminum

  - Splash Blocks - Precast concrete, standard size

  - Fasteners - Hidden straps @ 24” o.c.

- The following detail shall be used for the installation of the Downspout to EIFSS where EIFS system is present on a building.

- The following detail shall be used for the installation of the caulking at the window permiters to EIFSS.

EFIS Termination at the Window Header

Typical EFIS Termination at Window Jamb

Typical EFIS Termination at Window Sill

EXTERIOR PAINTING

- Painting of all exposed wood to include spot primer (unless full primer is specified by the Paint Manufacturer) and one (1) coat of paint, color to be selected by Owner. Bodies of buildings shall receive flat or satin paint and all doors, trim and previously painted exposed masonry shall receive semi-gloss paint.  Paint shall be applied with spray and shall be applied in sufficient quantities and in a manner consistent with “paint to cover” application. Paint sample to be provided for approval. Caulking all joints, perimeter of all openings such as windows, doors, vents, grills, meter/electric/cable/telephone boxes and nail heads.  Caulking shall be applied consistent with Section 07900.  Submittals must be made for approval.

- Properties with LP, T1-11 or Masonite Siding: Painting of all siding to include wire brush the bottom edge of each individual siding section.  Spot prime all brushed edges, and paint with one (1) coat of paint, color to be selected by Owner.  Paint shall be applied with spray and shall be applied in sufficient quantities and in a manner consistent with “paint to cover” application. Paint sample to be provided for approval. Caulking all joints and nail heads.  Caulking shall be applied consistent with Section 07900.  Submittals must be made for approval.

- Contractor to take moisture readings of all siding and wood prior to application of paint. A minimum of 10 readings per face (5 upper, 5 lower) must be taken on each building to receive paint. Painting shall be performed when the moisture contents are below 15% or at a range approved by the paint manufacturer’s representative.  All records of Moisture readings shall be maintained by the contractor and copies submitted to the Owner.

- The general contractor shall repaint all previously painted surfaces on of all the structures present at the community.  This includes all previously painted surfaces in all the buildings, the Pool House and the auxiliary structures such as mail kiosks, laundry and pool facilities, pool and property fences, trash enclosures, traffic sign posts, light posts, directional signs, previously painted gutters and downspouts, meter boxes and other accessories.

- The contract requires a 3-year (or 5-year where specified) paint manufacturer’s warranty against, labor, material and workmanship, defects. Contractor is responsible for the coordination of all inspections during the project and submission of the warranty documents to the Owner.

- All electrical boxes, gas boxes and lintels, if any, shall be painted whether previously painted or not.

- All wax bleed areas on trim and decks shall be primed with an oil-based primer per the paint manufacturer’s specifications.

- All corroded metal, whether previously painted or not, shall be prepared by using a wire brush on a drill motor and grinding surface rust from visible surfaces.  All bare surfaces shall be primed with a cold galvanizing compound such as “Galvalume” or equal.

- The contractor shall caulk seal all openings, including but not limited to windows, doors, sliding glass doors, vents, nail penetrations or joints in trim and in between trim and siding, and siding and brick.

- Repair any damaged siding, through replacement or re-nailing.

- Re-secure, replace, repairs as necessary all loose metal soffits, if any, by welding to supports.

- Re-secure all loose gutters and downspouts, and vents, including dryer and exhaust vents.

- Power-wash all maintenance free siding including but not limited to vinyl, brick, aluminum and roofing shingles.  Surfaces must be washed down with a solution of Trisodium Phosphate (TSP, water softener, Jomax), soap detergent and water.  Mixture of solution shall be manufacturer recommendation.  Power wash and clean all gutters and downspouts as part of this project.

- Prior to power washing the contractor shall bring to the attention of the Property Manager any broken vents, screens, lights, etc that are attached to the building surface. Damages must be signed off by the Property Manager and supplied to the Project Manager.  Otherwise the contractor shall be responsible for the replacement of vents, screens, etc that is damaged.

- Contractor shall employ only those persons who are skilled, experienced and regularly employed in the installation of the materials specified for this work.

- Materials shall be as specified or an approved equal.  Contractors wishing to use an alternate material must make a submittal in writing to the Owner and receive written authorization for using an alternate product.  The contractor shall be fully responsible for proving product equality to the Owners satisfaction.  Work completed utilizing alternate materials, without the Owners written authorization may be rejected by the Owner at their discretion.

- All surfaces shall be prepared to receive paint in strict accordance with the paint manufacturer's specifications and application guidelines.  The contractor will review the manufacturer specifications for application and surface preparation with the manufacturer representative; the Owner’s Construction Manager and the onsite property manager prior to the start of painting operations.  No painting or preparation shall take place prior to this pre painting conference.  The cost, if any, to have a manufacturers representative attend this meeting shall be included in the contractors bid.

- Completely remove all loose, failed, cracked and peeling paint, caulk, rust, mildew, grease, oil or other foreign materials from surfaces, scraping or sanding as required to achieve a sound substrate.  Sanding, scraping, washing or water blasting as required to achieve the desired results shall achieve this.  Under no circumstances what so ever shall any surface be painted that has not been property prepared.

- Remove all loose, cracked and failing caulking material.  All joints, butt, mitered, etc. will be caulked as necessary.  Areas around windows and doors shall be thoroughly caulked to prevent water penetration.  All holes, cracks, etc. in wall surfaces shall be prepared to match existing surface including existing nail holes.  Acceptable caulking materials shall be a polyurethane-based caulk. All caulking and sealant materials shall be completely compatible with the paints specified for adjoining surfaces.  No non-paintable caulking or sealant shall be used.  All caulking and sealant shall be installed as per the manufacturer’s specifications and in a good workmanship like manner.

- All protection of adjacent surfaces, including glass, brick, vinyl or aluminum siding, any unpainted surface, any painted surface of a different color, brick, fences, etc. is the complete responsibility of the Contractor and is to be considered included in the scope of work for this project.

- All debris, including excess paint and clean up/wash up materials, must be disposed of daily.  All such disposal shall be the complete responsibility of the Contractor and shall be performed in such a fashion as to be in compliance with all applicable local, state and federal codes, statutes, regulations, and guidelines.

- All surfaces to be painted shall receive sufficient coats of the appropriate finish and color to meet with the Paint Manufacturers standards and guidelines.

- No painted surface shall receive a second coat of paint until the first coat is completely dry, has been inspected by the Owners designated representative, and has been accepted by the Owner.

- Contractor may apply paints with rollers or brushes.  Airless spray equipment may be used to apply coatings providing:

- All surfaces not receiving that color or type of paint are masked and protected from over spray;

- All adjacent buildings and property, Including automobiles, etc., are protected from over spray.

- Contractor assumes all responsibility for any and all claims for loss or damage resulting from the spray operations.

21.	Contractor shall dismantle, disassemble, remove or make safe all ladders, scaffolding or other construction equipment when it is not specifically in use by the contractor's employees.  No ladders shall be left unattended at any time.

# General Notes

- The Work Hours shall be as follows:

Monday through Friday 8:00 AM until 6:00PM or dark whichever earlier.

Saturday 9:00AM to 5:00PM

Sunday and legal holidays no work shall be performed.

- The contractor’s onsite superintendent shall advise the Owner’s onsite Property Manager each morning on the progress anticipated for each day.

- Contractor agrees to submit a weekly progress report outlining the specific work completed during a given week, rain days encountered and proposed work schedule for the following week.  The progress report shall be send by facsimile transmission or e-mail to the following:

(Insert name, title and email address here)

From time to time Property Assignments and Project Manager Regions may change or overlap. The contractor will be notified who will be the designated project manager for the specific region/project prior to the start of the project.

- All work is to be coordinated with the Property Manager.  All movement of men and material shall be coordinated with the Property Manager.

- This facility will be in use during construction.  Extra care shall be taken to protect the general public.  Fences, barricades, etc., as may be prudent or reasonable to protect both the workers and the general public shall be placed around all work areas work areas.  All such protection, as well as any other safety equipment of any type, shall be furnished, installed and maintained by the Contract as a part of his work.

- All work shall meet or exceed all applicable codes.  The Contractor shall comply with all applicable municipal, state and federal regulations. The Owner reserves the right to stop the work at any time when a hazardous condition exists or is discovered. Work shall not resume until such time that the Owner is satisfied that the hazardous condition has been eliminated or secured.   The Owner shall assume no liability, financial or otherwise, for any consequences or losses that the Contractor may incur due to work stoppages.  The Contractor shall indemnify the Owner against any and all costs or consequences due to the Contractor’s failure to comply with any safety requirements

- The sites shall be cleaned daily into trash receptacles provided and paid for by the Contractor and located by the Owner.  At completion of the work, Contractor shall remove all waste materials and rubbish from and about the Project, as well as his tools, construction equipment, machinery and surplus materials, except those specifically required by the Contract Documents to be left for Owner's maintenance.  If Contractor or any Subcontractor fails to keep the project clean or to clean up prior to Date of Substantial Completion, the Owner may do so and the full cost for such clean up the responsibility of the Contractor.

- The Contractor shall supply all temporary utility connections to points approved by the Owner. These temporary utilities shall include legal temporary sanitary facilities for use by the contractor's workforce and visitors. These facilities shall be maintained by the contractor as to comply with accepted industry standards for such facilities. These facilities shall be located on the property in a location approved by the onsite property manager.  Installation, maintenance and removal of all materials required for the use of temporary facilities and utilities shall be the sole responsibility of the Contractor.

- The Contractor shall coordinate on-site storage of material with the Property Manager prior to delivery.  All material shall be new and free from any defects.

- Contractor shall properly store and remove volatile and toxic waste in covered metal containers from project site daily. Contractor shall not allow any volatile or toxic wastes to accumulate on project site.  Contractor shall provide adequate ventilation during use of volatile or toxic substances.

- Burning or burying of waste materials or rubbish on Project site is prohibited.

- Disposal of volatile or non-volatile wastes, such as mineral spirits, oil, paint thinner, oil or latex paint, or wash out residue from the any painting tools or equipment in storm or sanitary drains, on pavements, in gutters or downspouts or anywhere on project site is prohibited.  The Contractor shall be fully responsible for any and all costs connected with the identification or cleanup of any wastes improperly disposed of by the Contractor or any of the Contractor’s subcontractors.

- Disposal of waste or cleaning materials, which contain materials harmful to plant growth, on Project site, is prohibited.  Contractor shall clean up materials, which are accidentally spilled as quickly as possible.  Contractor shall be fully responsible for the remediation of any contaminated soils and the replacement of any damaged landscaping or plants.

- Contractor shall be responsible to legally dispose of all waste materials, rubbish, volatile materials, and cleaning materials off project site.

- Contractor shall clean all finished surfaces in accord with manufacturer's product data and requirements specified in trade section, prior to the Date of Substantial Completion.  All general and specific cleaning shall be performed prior to Contractor or Subcontractor's request that the project or portion thereof be inspected for Substantial Completion.

- Replace, patch and touch up marred surfaces to match adjacent finishes.  Replace materials, which cannot be repaired or patched.

- Clean disturbed areas of project site of debris:

a: remove all paint scrapings, nails and discarded construction material from all landscaping and resident balconies, decks and all other common areas.

b: broom clean all concrete and/or paved surfaces.

- The property management office shall designate all contractors parking.  If necessary, violators will be towed at the contractor’s expense.

- The Project Manager and the property management office shall be furnished with a phone number where contractors can be reached and/or beeper number of the supervisor whenever contractor or his sub contractors are on the job site.

- The property management office shall be notified by 4:00 p.m. Thursday for weekend scheduling.

- Any damage to the unit, buildings, or the surrounding grounds, caused by this contractor's own forces or subcontractor's forces are the responsibility of this contractor.

- Tenants shall be given ample warning and time when work will take place on the building they are located in.  Notices to tenants notifying them that work will be taking place shall be provided for and distributed by the Property.  Such notification shall be made at least 24 hours in advance of the actual work taking place. Contractor is responsible to notify the Property Manager 48 hours in advance of the actual work taking place.

- Contractor's employees shall not disturb any tenants, congregate in common areas in or around the buildings or outside lobby doors.

- The contractor shall require all of his employees, subcontractors and subcontractor's employees to comply with the following:

- All construction personnel shall conduct themselves in a professional manner and use all reasonable care not to disturb any tenant or visitor.

- The consumption of any alcoholic beverages on the work site shall be prohibited at any time.

- The use or possession of any illegal substance or the abuse of any drug shall be prohibited at any time.

- All workers shall be appropriately clothed at all times.  This shall include the wearing of shirts, long pants and appropriate footwear.  Sneakers shall not be acceptable footwear.

- Any individual who violates these regulations may be banned from the worksite permanently.

- Repeated violation of these requirements by employees of the Contractor may result in the project being stopped or the termination of the contract.

SECTION 01510 TEMPORARY REQUIREMENTS

- Keep a set of specifications on site at all times.

A.	ELECTRICITY AND WATER

- Electric and water service is available at the site.  The Contractor shall have the right to use such public utility services in the performance of the work here under.  Any connections or tie-ins to existing services at the site shall be the sole responsibility of the Contractor.  Prior to the start of the projects the Contractor shall notify Owner in writing of power requirements required for completion of project.

B.	PUMPING AND DRAINING

- Keep working and storage areas free from water that could cause damage or interfere with work.

C.	TELEPHONE

- The contractor shall have a telephone on site at all times.

D.	CLEANING

- Remove temporary requirements when no longer needed.  Clean and repair areas occupied by temporary work on a daily basis.

E.	DUST BARRIERS/FENCING

- Contractor is to note that all excavations require protective fencing to limit access to open trenches.  Access to construction area is to be limited at all times.

F.	PROJECT SIGN

None will be allowed.

- G.	SECURITY

Contractor is responsible for security of all his equipment and damage to/or caused by any and all causes or persons.

DIVISION 4 BRICKWORK

SECTION 04210 BRICKWORK

PART 1 GENERAL

A.	QUALITY:

Meet  requirements  and  recommendations  of  applicable  portions  of standards listed.

American Society for Testing Materials - ASTM

Structural Clay Products Institute - SCPI

B.	SUBMITTALS:

Submit samples of brick for the Project Manager’s approva1. Sample to be 4' x 4' brick pane1. Free-standing, not a part of the building project.  Match existing brick texture and color.

C.	JOB CONDITIONS:

Freezing Weather: No masonry shall be laid when temperature of outside air is below 40@ F., unless protective means (approved by Architect) are provided. Protection shall consist of heating and maintaining temperature of masonry for 48 hours when using Type `S' mortar. This period may be reduced to 24 hours, if high-early-strength mortar is used.

PART 2 PRODUCTS

A.	MATERIALS:

1. Face Brick: ASTM C2l6-75 Grade SW match existing.

2. Flashing: Flexible plastic, Nervastrol (at sill and heads)

3. Mortar: Type S

4. Brick Sealer: clear non-staining, submit mfr’s data

NOTE:	Brick delivered to the job site shall not have manufacturer's name/logo  imprinted  on  any  finished  surfaces.  Entire  lot containing such imprints shall be rejected.

PART 3 EXECUTION

A.	LAYING BRICK:

1. Lay brick plumb,  level and true to line in running bond, unless otherwise indicated.

2. Finish joints to remain exposed with 1/4" to 3/8" concave joint.

3. Brick having absorption rates (ASTM C 67) in excess of 0.024 oz. per square per inch per minute,  shall be wetted so rate of absorption is not exceeded.

4. Lay  brick  in  full  bed of mortar with head and edge joints completely filled.

5. Cut brick, where necessary for fitting or bonding, with a  masonry saw.

6. Provide weep joints, at approximately 4'  o.c. Keep weeps above flashing.

7. Brick base to be carefully protected during construction  (cover with sand); any discolored or chipped brick shall be  replaced at contractors expense.

8. Brick ties shall not support more than 2 sq. ft. maximum.

B.	JOINING OF WORK:

Where fresh masonry joins partially or totally set masonry,  the masonry shall be cleaned and lightly wetted.   All loose brick and mortar shall be removed.   If necessary for construction purposes to "stop off", a horizontal run, this shall be done only by racking back in each course.  Toothing will not be permitted.

C.	TOOLING JOINTS:

When joints have become "thumbprint" hard, they shall be tooled such that compressing and sealing of the joint occurs, tool head joints first.  Joints shall be concave tooled.

D.	EMBEDDED ITEMS:

1.  Notify all trades with embedded items in ample time.

2.  Build in flashing, sleeves, anchors, clips and accessories as  	work progresses.

3.  Install loose lintels, as indicated, in full beds of mortar.

4.  Fill voids at metal with mortar.

5.  Lay brick to include metal wall ties and joint reinforcing from back-up.

E.	CLEANING:

1. All holes shall be pointed and defective joints shall be cut and re-jointed.

2. Exposed masonry shall be protected against staining by wall coverings and excess mortar shall be wiped off as work progresses.

3. All brick surfaces shall be free of efflorescence.   Cleaning, sandblasting, and pointing at completion of masonry work shall be preformed, with defective joints or brick cut out and replaced.

4. All exposed masonry brickwork shall be thoroughly cleaned.  Before applying any cleaning agent to the entire wall,  it shall be applied to sample area 20 square feet in a location approved by the Architect. No further cleaning work may proceed until sample area shall be thoroughly wetted with clear water and then scrubbed with Sure-Klean as per manufacturer's directions and followed immediately by a thorough rinsing with clear water.   All sash, metal lintels and other corrodible parts shall be thoroughly protected during cleaning.

F.	EXPANSION JOINTS:

Unless otherwise noted, expansion joints shall be located at a minimum of 25'-0" o.c.   The exact locations shall be verified with the Project Manager.  Care shall be taken to insure that the joints are free of mortar or debris.

DIVISION 6 WOOD AND PLASTICS

SECTION 06100 ROUGH CARPENTRY

PART 1 GENERAL

A.	SCOPE:

1.	All new wood shall be six sided primed prior to installation.

2.	All pressure treated materials shall be pressure treated kiln dried lumber and shall be suitable for immediate painting or staining

3.	Items to be included, but are not limited to:

a.	Simpson connectors

b.	Framing & Blocking

c.	Hydraulic Jacks and shoring

- B.	DELIVERY AND STORAGE:

- Deliver and store lumber on sills and cover for protection.  Ensure ventilation and drainage; protect against weather and damage.  Keep all materials clearly identified, with all grade marks legible.  Keep all damaged material clearly identified and separated.  Store the material so as to prevent inadvertent use.  Protect all installed work and materials of all other trades.

- C.	QUALITY:

- 1.	All lumber used structurally shall be graded and marked with grade and trade mark of a lumber grading organization approved by Engineer, except that a certificate of grade from such a grading organization may be accepted in lieu of grade and trade marks when approved by Engineer.  Trademark of manufacturer shall also appear on each piece.

- 2.	Each piece of plywood used structurally shall carry the American Plywood Association trade mark.

- 3.	Grading Rules: Conform with all applicable requirements of American Lumber	Standards “Simplified Practice Recommendation R-16” and to grading rules of 		manufacturer’s association under whose rules the lumber is produced.

- 4.	Conform to all requirements as they apply:

a.	U. S. Department of Commerce Product Standards

b.	American Wood Preservers Association Standards Southern Pine Association

c.	Douglas Fir Plywood Association West Coast Lumber Inspection Bureau

PART 2 PRODUCTS

A.	WOOD PRODUCTS:

1.	All framing lumber shall be stress-grade; all sides surfaced.  Grades as follows:

USE			GRADE		SPECIES  		Fb(psi)

a.	Structural &Select structural 			Douglas Fir or		1200

Joist			#2 KD			Southern Pine

b.	General	Load Bearing & Non			Douglas Fir		1000

Load Bearing 	framing & Studs			Lodgepole Pine

construction grade

c.	Furring &		Standard		Douglas Fir or		850

Blocking		Utility			Southern Pine		550

2.	Plywood shall be graded per Product Standard PS-1 established by the U. S. Dept. of Commerce.  The following grades are those of the American Plywood Association.  Each sheet shall carry the trademark of the APA.

a.	Roof, Wall and Floor Sheathing:  C-D plywood with exterior waterproof glue.

b.	Roof Plywood: 5/8", 24/0 plywood, C-D faces, unsanded, exterior waterproof glue, carrying marks of the American Plywood Association.

c.	Exterior Concrete Deck:  Douglas Fir, DD faces, pressure treated.

B.	PRESERVATIVE TREATMENT:

1.	All wood sillplates, ledgers, posts, and stringers in direct contact with concrete or Earth shall be pressure treated.  Treated wood shall be dried after treatment to a moisture content of less than 18%.  Submit certification prior to use on job.

2.	All blocking, studs, joists, posts, etc. within 12" of concrete shall be treated after sawing by soaking ends in solution for not less than 15 minutes.

3.	Pressure-Preserved Plywood: Treated plywood, where required, shall be pressure treated as per American Wood Preservers Association Standard C-9 as required for soil or water contact above ground exposure.  Sheets shall be APA grade marked, C-D Grade or Better, and made with exterior glue.  Treated ply shall be identified with AWPA quality mark. (All plywood treated with preservative shall be dried after treatment to moisture content of 18% or less.)

C.	Sheathing paper shall be 15 lb. asphalt impregnated building paper as required by local codes.

PART 3 EXECUTION

A.	COORDINATION:

Coordinate work with all other trades and do all cutting and patching required to accommodate their work.  Protect all adjacent work.

B.	FRAMING:

1.	Fit closely;  set accurately to required lines and levels and secure rigidly in place.  Set horizontal and inclined members with crown edge up.  Do not cut,  notch or bore structural members without specific approva1.  Reinforce cut members as directed.  Bolt, nail and spike thoroughly with not less than sizes and quantities  indicated or as required by local building codes.  Structural members  shall  provide  full contact at all bearing surfaces.

C.	TERMITE CONTROL AND DECAY PREVENTION:

Remove all wood, including form lumber, scrap lumber, shavings and sawdust in contact with ground.  Leave no wood buried in any fill or backfill.

D.	CONNECTIONS:

1.	Nails:  Bright common wire nails, galvanized for exterior work.  Sub-drill where 	necessary to splitting.

2.	Bolts: Drill bolt holes 1/32 inch larger than bolt diameter. Use square plate or malleable iron washers under heads and nut where they bear against wood.  Re-tighten bolts immediately prior to concealing with finish work.  Re-tighten exposed bolts immediately prior to final inspection.

3.	Lag Screws and Screws:  Sub-drill, use square plate or malleable iron washer under lag screw heads when they bear on wood.

4.	Fabricated Connections: Sheet metal galvanized of size and type required for proper connection as manufactured by Simpson or approved equal.

E.	SHEATHING PAPER: Shall be 15 pound asphalt impregnated building paper.

1.	Install sheathing paper under exterior wall sheathing, under all cement or plaster; lay horizontally starting at bottom; lap edges and ends 6 inches and extend in back of 	window casings and other finish work.  All corners shall have a continuous section installed vertically after installation of horizontal runs.

06180 – FIBER-CEMENT SIDING

A.	General

1.	Applicability – The General Conditions, Supplementary General Conditions and the applicable requirements of Division 1 apply to each and every contract sub-contractor or other persons supplying materials and labor for this project.

2.	Quality

a.	Meet the recommendations and requirements of the applicable portions of the standards listed.

1)	National Evaluation Service – NES

2)	American Society for Testing and Materials – ASTM

3)	International Congress of Building Officials – ICBO

4)	Texas Department of Insurance

5)	U.S. Department of Housing and Urban Development – HUD

3.	Product Handling

a.	The siding shall be stacked on edge or laid flat on a smooth, level surface.  Edges and corners shall be protected from damage or chipping.  The siding shall be stored under cover and kept dry until installation.  No siding shall be installed in a damp or wet condition.

4.	Job Conditions

a.	Siding shall not be installed in rain or inclement conditions.

B.	Products

1.	Materials:

James Hardie Building Products

26300 La Alameda, Suite 250

Mission Viejo, CA 92691

2.	Siding:

5/16” Hardi-plank  8” maximum exposure 12 foot lengths.  Pattern as selected by owner.

3.	Panels:

5/16” Hardi-panel  4’ x 8’ sheets.  Patterns as selected by owner.

4.	Fasteners:

Hot dip galvanized, or electroplated zinc (or stainless steel in coastal areas) with 3/8”  head.

6d – 2” long ring shank nails at single planks

6d – 2” long common nails at overlapping planks

6d – 2” long common nails at 6” on center, edge and field.

C.	Execution

1.	Install over stud wall framing spaced a maximum 24” o.c., over GyProc or Dens-Glass sheathing with Tyvek Weatherization Barrier Wrap.  Install a 6 mil poly wrap (12” wide minimum)at all corners.

2.	Install ¼” lath starter strip at base of wall for Hardi-plank siding.

3.	Install 24 gauge GI flashing at window and door heads.  Flashing shall be 1½” x width of trim, formed sheet metal with a drip edge and caulk sealed to panel or plank.

4.	Vertical joints shall butt over studs with a minimum 48” space between successive rows of splices with no splice located closer than 72” from the building corner or an opening.

5.	Metal joiners shall not be used.

6.	All inside corners shall have a 2 x 2 installed and caulk sealed to planks.

7.	A minimum clearance of 3/8” is required from all edges and 1” from top of plank for all nails.

8.	All butt joints or exposed joints shall be caulk sealed with an acrylic latex caulk or as recommended by the manufacturer.  No caulking shall be allowed to remain on finished surface of siding.

- Siding and trim shall be pre-primed or primed in place and all damage to primer touched up prior to applying finish coats.

- Siding blocks consisting of 1-1/2” thick lumber shall be installed beneath all sconce-type light fixtures prior to siding installation.  The electrical contractor shall leave a loop of romex near the fixture location so that the siding block can be bored to pull the romex through for connection to the light fixture utilizing a ‘pancake’ type j-box.  Siding blocks exposed to weather shall have a galvanized sheet metal ‘zee’ flashing installed overhead lapping behind the siding above.  Coordinate siding block locations with Electrical Contractor.

SECTION 07900 - JOINT SEALERS

PART 1 GENERAL

1.01	 RELATED DOCUMENTS

A. 	Provisions established within the General and Supplementary General Conditions of the Contract, Division 1 - General Requirements, and the Drawings are collectively applicable to this Section.

1.02	RELATED WORK

A.	Section 06200	Finish Carpentry

B.	Section 09251	Gypsum Drywall

C.	Section 09900 Painting

1.03	Definitions

A.	Sealant

1.	Refers to exterior joints exposed to the weather or interior joints exposed to moisture. When "sealant" is used in an outside joint in aluminum or steel frames, "sealant" shall be required on the inside joint as well.

B.	Caulking

1.	Refers to interior joints not normally exposed to weather or moisture conditions.

1.04	QUALITY ASSURANCE

A.	Manufacturer: Company specializing in manufacturing the products specified in this Section with minimum 5 years documented experience.

B.	Applicator: Company specializing in applying the work of this Section with minimum 3 years documented experience and approved by sealant manufacturer.

C.	Conform to Sealant and Waterproofers Institute requirements for materials and installation.

D.	The manufacturer of caulking and sealant products shall provide factory representatives who shall conduct periodic on-site checks of the Work to determine compliance with the manufacturer's installation requirements

1.05	SUBMITTALS

A.	Submit product data under provisions of Section 01300.

B.	Submit product data indicating sealant chemical characteristics performance criteria, limitations, color samples and availability and application instructions.

C.	Submit samples under provisions of Section 01300.

D.	Submit two samples 1/4 inch diameter x 4 inches in size illustrating color selections available.

E.	Submit manufacturer's certificate under provisions of Section 01405 that products meet or exceed specified requirements.

F. 	Identify each product's use and location.

1.06	GUARANTEE

A.	General

1.   	Provide warranty under provisions of Section 01740.

2.	The Contractor, sub-contractor and product manufacturer shall jointly guarantee the Work against defects in materials and workmanship for a period of five years from the date of Substantial Completion.

3.	Include coverage of installed sealants and accessories which fail to achieve air tight and watertight seal, exhibit loss of adhesion or cohesion, or do not cure.

B.	Conditions

1.	Repair and/or replace such defective Work, and other damaged work, which becomes defective during the guarantee period at no additional cost to the Owner.

- This guarantee will not apply to defects caused by failure to Work of other trades, if such defects were impossible to detect at the time of installation.

1.07	EVIRONMENTAL REQUIREMENTS

A.	Do not install solvent curing sealants in enclosed building spaces.

B.	Maintain temperature and humidity recommended by the sealant
	manufacturer during and after installation.

1.08	SEQUENCING AND SCHEDULING

A.	Coordinate the work of this Section with all Sections referencing this
	Section.

1.09	DELIVERY AND STORAGE

A.	Deliver materials in unopened containers as packaged by manufacturer.
	Store in a manner to protect materials from weather.

PART 2		MATERIALS

2.01	ACCEPTABLE MANUFACTURERS

A.	Pecora Chemical Corp.

B.	Tremco Manufacturing Co.

C.	ICI Dulux Paint Company

D.	General Electric Silicone Products Division

E.	Dow Corning Corp.

F.	Substitutions: Submit in accordance with Section 01630.
2.02	MATERIALS

A.	Polyurethane (Type 1 Sealant):

1.	One part conforming to FS TT-S-00227E, Class A,
		Type i (self-leveling) and ASTM-920.

2.    	Color: As selected by Architect

3.	Acceptable products:

a.    Urexpan NR-210, Pecora

- Sonolastic SL-1, Sonneborn

B.	Silicone, General Purpose (Type 2 Sealant):

1.	One-part rubber based silicone conforming to FS TT-S-
	001543, Class A, Type II and ASTM C-920.

2.   	Color: Match substrate.

3.	Acceptable products:

- 790 Building Sealant, Dow Corning

- Silpruf, General Electric

- Spectrum 1, Tremco

- ICI-Dulux  955-clear

C.	Silicone, Paintable (Type 3 Sealant):

1.    	One-part silicone rubber sealant for interior use.

2.	Acceptable product: Silicone Rubber Sealant, Paintable
	Dow Corning.

D.	Silicone, Sanitary (Type 4 Sealant):

1.	One-part conforming to FS TT-S-001543, F.D.A. Regulation
	21 CFR177.2600, and F.D.A. Food Additive Regulation 121.2514. mildew resistance. Paintable

- Colour: White

- Acceptable Products

  - (1) 786 Silicone Rubber Sealant, Dow Corning

  - SCS1702, General Electric

E.	Acrylic Latex (Type 5 Sealant):

1.	One-part, non-sag acrylic latex, meeting requirements of ASTM C834.

2.	Acceptable products

a. AC-20, Pecora.
	b. Sonolac, Sonneborn-Contech, Inc.
	c. Acylic Latex 834 Caulk, Tremco
	d. Sherwin Williams C-850A

F.	Acrylic Sealant (type 6 Sealant):

1.	One-part, non-sag, acrylic polymeric conforming to FS TT-S-00230.

2.	Acceptable products

a. Unicrylic 60, Pecora

b. Mono, Tremco Manufacturing Co.

G.	Polyurethane (Type 7 Sealant):

1.	One-part conforming to FS TT-S-00227E, Class A, Type
	II (non-sag) and ASTM-920

2.    	Color: As selected by Architect.

3.	Acceptable products

- Dynatrol I, Pecora

- Sonolastic NP-1, Sonneborn.

F.	Foamed-in-place (Type 8 Sealant):

1. Polycell.

2.03	ACCESSORIES

A.	Joint Cleaner: Non-corrosive and non-staining type, recommended by sealant manufacturer; compatible with joint forming materials.

B.	Joint Backing: ANSI/D1056 and D1565 round. In vertical joints use closed cell polyethylene foam rod; oversized 30 to 50 percent larger than joint width. In horizontal joints, use solid neoprene or butyl rubber, Shore A hardness of 70.

1.	Rods and tape as required shall be in sizes and types as recommended by the manufacturer of the caulking and sealants and shall be compatible with the compounds as specified above.

2.	Joint backing shall be installed in all joints to receive sealants.

3.	Backing shall be placed so that the sealant depth is approximately one-

half the joint width.

4.	In joints not of sufficient depth to allow backing, install bond breaker tape

at the back of the joints.

2.04 	PRIMERS

A.	The type of primer shall be as recommended by the manufacturer of the caulking or sealant and be completely compatible with the compounds as specified above.

PART 3 EXECUTION

3.01 GENERAL

A.	The Work shall be performed in strict accordance with the manufacturer's written requirements.

B.	The work shall be performed by experienced mechanics, skilled in the execution of the type work required and in application of the specified materials.

C.	Deliver the materials to the job site in original containers with the manufacturer's name and brand clearly marked thereon.

D.	When joints that are to be sealed do not have built-in stops to prevent the depth of the compound from exceeding 1/2", pack the joint with back-up materials of the correct type and depth as necessary to provide a minimum of 3~8" and a maximum of 1/2" depth of compound.

E.	If, prior to beginning the work, the Contractor does not notify the Architect in writing of any proposed changes, it is assumed that he agrees that the materials as specified will produce the desired results and that he will furnish the required guarantee.

3.02	EXAMINATION

A.	Verify that surfaces and joint openings are ready to receive work and field measurements are as shown on Drawings and recommended by the manufacturer.

B.	Beginning of installation means installer accepts existing substrate.

3.03 	PREPARATION

A.	Where weather moulds, staff beads, etc., do not form an integral part of the frames to be sealed, but are removable, remove same prior to sealing, execute sealant, replace moulds, etc., and point.

B.	Where pre-molded expansion joint fillers have been installed with temporary wood or other type fillers which remain in the joints above expansion joint fillers to allow for final sealing, remove such temporary fillers, clean the joint thoroughly prior to sealing.

C.	Clean all joints, etc., in accordance with manufacturer's instructions, that are to be caulked or sealed prior to executing the work.

D.	Remove loose materials and foreign matter  which might impair adhesion of sealant.

E.	Verify that joint backing and release tapes are compatible with sealant.

F.	Perform preparation in accordance with ASTM C804 for solvent release sealants, C790 for latex base sealants, C919 for acoustical applications, and C962 for elastomeric sealants.

G.	Protect elements surrounding the work of this Section from damage or disfiguration.

  - PRIMING

A.	When the conditions of the joints so require, or when the types of materials used adjacent to the joints so require, or when the compound manufacturer's recommendations so require, clean and prime the joints before starting the caulking and/or sealing. Execute the priming operations in strict accordance with the manufacturer's instructions.

3.06	APPLICATION

A. 	Install sealants in accordance with manufacturer's instructions.

B.	Measure joint dimensions and size materials to achieve required width/depth

ratios.

C.	Install joint backing (if required) to achieve a neck dimension no greater than 1/3 the joint width.

D.	Apply sealant within recommended application temperature ranges. Consult manufacturer when sealant cannot be applied within these temperature ranges.

E,	Apply caulking and sealant materials under pressure to fill the joint completely.

F,	Install sealant free of air pockets, foreign embedded matter, ridges, and sags.

G.	Tool the joint surface to compress the compound into the joint fully.

H. 	Tool joints concave. Sealant shall achieve a firm skin before surface coating is applied.

I.	On fire retardant putty and foam, install in such a manner as to maintain hourly rating of element which is being penetrated.

3.07	JOINT BACKING

A.	Joint backing shall be installed in all joints to receive sealants. Packing shall be sized to require 30% to 50% compression upon insertion and shall be placed so that the sealant depth is approximately one-half the joint width. In joints not of sufficient depth to allow backing, install bond breaker tape at the back of the joint.

3.08 THRESHOLDS

A.	Place all exterior door thresholds in a full bed of sealant during the setting procedures.

3.09 CLEAN UP

A.	Clean all adjacent surfaces free of caulking and sealants and clean all work of other trades that has, in any way, been soiled by these operations.

B,	Repair or replace defaced or disfigured finishes caused by work of this Section.

C.	The finished work shall be left in a neat and clean condition.

3.10	PROTECTION OF FINISHED WORK

A.    Protect finished installation under provisions of Section 01500.

B.     Protect sealants until cured.

3.11 SCHEDULE

SEALANT TYPE	APPLICATION

1		a.	Horizontal control and expansion joints in concrete flooring and pedestrian paving and at junctures between these materials and other adjacent materials.

2.	b.	Sealing of joints between plumbing fixtures and substrates and between plastic laminate splashes and adjacent tops and walls.

3		a.	General caulking as part of interior painting in joints

subject to movement.

4		a.	Sealing joints between countertops and substrates in kitchen and elsewhere which may be in contact with food.

5.			a.	General caulking as part of interior painting.

b.	At interior wood door frames, shelving, misc., trim, and base where it meets sheetrock.

c.	At all interior wood base where it meets vinyl flooring;

d.	At all interior wood to wood conditions, i.e. door stops, butt joints, & trim to frames;

e.	At all interior window frames to sheetrock returns.

f.	Caulking at cabinets, toilets, etc. to be applied by those trades.

6			a.	Setting sill plates to slabs.

7			a.	Sealing of joints in exterior wood siding and soffit work.

b.	Control joints in masonry work and at juncture between masonry work and adjacent materials.

C.	At all junctures of dissimilar materials and where specifically required by drawings;

d. 	At nail holes in exterior trim at eye level and near entry areas;

e. 	At all excessively countersunk nails;

8.	a.	Sealing around perimeter of all windows, doors, and other items penetrating exterior siding prior to installation of drywall.

END OF SECTION

DIVISION 9 FINISHES

SECTION 09900 PAINTING

PART 1 GENERAL

A.	The General Conditions, Supplementary General Conditions and the applicable requirements of Division 1 apply to each every contractor, sub-contractor or other persons supplying materials and labor for this project.

B.	WORK NOT INCLUDED:

1.	Copper, bronze, chromium plate, nickel, stainless steel, anodized aluminum lead and bright metals.

C.	The following specifications cover the complete painting and finishing of all exposed to view exterior surfaces.

1.	Louvers, grilles and registers.

2.	Vents, exhaust fans and flashings.

3.	Doors, siding, balconies, pool fences, metal railings, fascia and soffits.

4.	Perimeter property fence and gates.

- Electrical boxes, transformers, meter enclosures and a/c disconnects

- All new wood shall be 6 side primed prior to installation.

D.	The contractor shall supply all labor, materials, tools, ladders, scaffolding and equipment necessary for the completion of this work according to the Specifications.

E.	The contractor shall refer to the Bidding Documents that form a part of the Contract and to the alternates that affect the work under this section. Examine the Specifications under other divisions and thoroughly familiarize himself with all provisions regarding their painting. Allowances for repainting existing components damaged during construction shall be made and included in the base bid. No change order will be issued for any area damaged during the construction process.

F.	QUALITY:

1.	Meet requirements and recommendations of applicable portions of standards listed.

a.	Paint and Decorating Contractors of America - PDCA

b.	Comply with ASTM Designation E84, E162, 62T

G.	SUBMITTALS:

1.	Submit samples of all painted surfaces for approval.

2.	Submit as per the requirements of General Conditions.

H.	PRODUCT HANDLING:

- Defaced Surfaces - At the completion of the work, the Contractor shall remove all spilled or splashed paint or overspray from surfaces that have been defaced.

- Material and Debris Removal - At completion of the work, the Contractor shall remove from the job side, all painting equipment, scaffolding, surplus materials and debris resulting from his work.

- Trash and Debris - The Contractor shall be responsible for keeping trash and debris from collecting or being spread across the job site during the course of the job. Oil and solvent soaked rags shall not be allowed to accumulate and they shall be removed from the job site every evening. Every precaution should be taken to avoid danger or fire.

- Specified Materials -

The preferred materials suppliers are ICI DULUX OR Sherwin Williams.  The owner has a national program set up with them.  Other paint manufacturer’s material will be considered based on quality of material and cost savings for the project. Bids must specify paint manufacturer if other than ICI DULUX or Sherwin Williams.

- All coating materials - Shall be delivered to the job site in the original and unopened containers, plainly marked with the proper designation of the product, as well as the name of the manufacturer. All coating materials at the job site shall be subject to inspection.

- Storage Conditions - All painting materials shall be stored in a clean, dry, well ventilated placed, protected from sparks, flame, direct rays of the sun, and excessive heat or cold. The Contractor shall be solely responsible for the protection and safety of the materials stored by him at the job site.

- Paint Additives - Paint additives are prohibited unless specified and approved by the coating manufacturer.

- Minimum Specifications - If instructions contained in these specifications, bid documents or the painting schedule are at variance with the paint manufacturer’s instructions or the applicable standards and codes published by SSPC or OSHA, surfaces shall be prepared and paint applied to meet the higher standard, as determined by the Project Manager.

- Resolution of Conflicts - The Contractor shall be responsible for requesting prompt clarification when instructions are lacking, conflicts occur in the specification and/or paint manufacturer’s literature, or the procedure specified is not clearly understood.

I.	JOB CONDITIONS:

- The painting Contractor is responsible for inspecting the work of others prior to the application of any painting or finishing material. If any surface to be finished cannot be put in proper condition for finishing by customary cleaning, sanding and puttying operations, the painting contractor will immediately notify the Owner in writing, and shall not proceed with this work until conditions have been corrected and are acceptable.

- When surface temperature is below 50 degrees F., do not apply paints, varnishes and special coatings, unless specified. Do not prime exteriors during frosty or rainy weather. No coating should be applied unless the surface is a minimum of 5 degrees above the dew point. This requirement also applies during curing of the coatings.

- Dew Point Chart

Ambient Air Temperature:

Degrees (Fahrenheit)

20     30     40     50     60     70     80     90     100     110

Relative

Humidity

90%			18     28     37     47     57     67     77     87      97      107     117

85%			17     26     36     45     55     65     75     84      95      104     113

80%			16     25     34     44     54     63     73     82      93      102     110

75%			15     24     33     42     52     62     71     80      91      100     108

70%                         	13     22     31     40     50     60     68     78      88        96     105

65%			12     20     29     38     47     57     66     76      85        93     103

60%   			11     19     27     36     45     55     64     73      83        92     101

55%			  6     15     23     31     40     50     59     67      77        89       98

50%			  4     13     21     29     37     47     56     64      73        82       91

Dew Point: Temperature at which moisture will condense on the surface.

Example:  If the ambient air temperature is 70 degrees F and the humidity is 65%, the due point is 57 degrees F and no coating should be applied unless the surface temperature is at least 62 degrees F.

- Before any surface is coated, it shall be cleaned carefully of all dust, dirt, grease, loose rust, mills scale, old paint, etc. Immediately prior to painting, the surface shall be inspected for compliance with the specified degree of surface preparation.

- Exterior painting shall only take place when good weather conditions prevail.  Painting shall not be undertaken when foggy or misty conditions or when rain or snow are imminent. Do not paint over surfaces containing excessive moisture.  Do not apply a coat over another coat until thoroughly dried. Follow recoat times on manufacturer’s specification data sheets.

- When substrate temperatures are high, care must be taken while applying the paint to prevent formation of voids, pin holes and bubbles due to the rapid evaporation of solvent.

- The painting contractor shall examine the surface to be painted for any existing embedded algae or other substance which could effect the bonding of the paint film and include proper preparation of the surface in the base bid.

K.	SUBSTITUTIONS AND EXTRAS:

- All materials used on the work shall be exactly as specified in brand and quality.  No claim by the painting contractor as to the unsuitability or unavailability of any material specified, or his unwillingness to use same, or his inability to produce first class work with same, will be entertained unless such claims are made in writing with his bid.

- The painting contractor shall not be entitled to payment in excess of the amount agreed upon in his contract for any extra work over and above that specified.

PART 2 - PRODUCTS

A.	MATERIALS:

1.	The contractor shall indicate the paint manufacturer proposed in his bid submittal package. The paint shall have a minimum 3 year manufacturer’s material warranty and the labor shall have a minimum 1 year warranty supplied by the manufacturer.

2.	The paint shall be applied to cover existing color.

3.  	Refer Part 1.H.4.

B.	PAINT COLOR:

- The owner will select and/or approve colors and determine the number of colors to be used on the job.

2.	Contractor shall prepare 4 sample panels showing finish and color using the materials as approved, a minimum of 10’ X20’.

PART 3 - EXECUTION

A.	PREPARATION:

- All surfaces shall be clear of all contamination by using pressure washers with a minimum of 1000 PSI nozzle to a maximum of 3000 PSI nozzle. Remove and treat all surfaces for algae and mildew. Use 4 parts water to 1 part Clorox. Let it soak for a minimum of 5 minutes, then power wash. Contractor is responsible for himself and his worker’s protection to include, goggles, rubber gloves and protective clothing. Protection is also to be provided for the surrounding areas.

- Use hand-held tools to remove all loose paint and other contaminants from the surface. Repair the surface by filling cracks, holes, etc. with an appropriate material.  Remove surface projections. Ensure that the surface is clean, free of oil or grease and dry before applying any paint.  Six side prime all exterior new wood.   Brush all delaminated areas of the siding with a primer as  recommended by the manufacturer.

- Clean floors and adjacent surfaces, as well as all surfaces to be painted, prior to painting. Areas where painting is to be done shall be broom clean and dust free.

- Touch up knots, pitch streaks and sappy spots with a recommended primer prior to painting.

- Prime all existing wood where paint has been removed due to the preparation or power washing method used on the project.

- Wash metal surfaces with mineral spirits to remove dirt, oil and grease prior to applying paint. Remove rust and scale by wire brushing or sanding clean before painting. Clean and touch up shop coats of paint that have become badly weathered, worn or marred with the primer specified.

- Clean galvanized metal thoroughly and apply recommended primer.

- Work to be finished with enamel shall be prepared  to allow adhesion and grab of finish coat per the manufacturer’s recommendations.

- Remove all loose caulking or any silicone caulking and replace with that specified.

- Areas that have wax bleed shall be coated with 1 coat of  primer per the manufacturer’s requirements.

- Staining woods need to be primed to prevent bleed through. Seal any knots in the surface of wood or plywood prior to painting. If knots are not sealed, they may ‘bleed’ and discolor the coating and the contractor will be responsible for any repainting as a result of bleed through or discoloration.

B. 	APPLICATION:

- Secure color schedules before applying paint or finish.

- Use materials only in accordance with the manufacturer’s directions.

- Use only approved materials which bear identifying labels on the containers.

- Use factory mixed colors, shades and tints; job mixing not permitted.

- Employ skilled craftsmen to ensure the very best workmanship. Quality workmanship is required. Materials shall be applied by craftsman experienced in the use of the specific product involved.

- Exterior Work: Paint all exposed-to-view or exposed-to-weather materials and trim.  Paint each type of 	material as per the manufacturers specifications

- Before proceeding with any painting, the Painting Contractor shall prepare the finish in a sample area, complete or in part, as directed by the Project Manager. Finish all areas or items in accordance with the specifications and in colors selected by the Owner and/or his representative; if such areas or items are not approved, finish other sample area. When approved, they shall serve as standards for workmanship, appearance, and materials approved for similar areas or items throughout this project.

- Shop coats and priming coats specified elsewhere shall be counted in the number of coats specified in this section.  Prime coats shall be compatible with the color of the finish coat, but shall be a different shade.

- Paint interior surfaces  which are part of exterior materials per ‘Exterior Work’.

- Apply all materials under adequate illumination, spread evenly and flow on smoothly without run or sags.

- All coats must be thoroughly dry before applying succeeding coats.

- All work where a coat of material has been applied must be inspected and approved by Owner’s representative before the application of the succeeding specified coat, otherwise, no credit for the coat applied will be given and the Contractor shall then assume the responsibility and recoat the work in question.

C.	CLEANING:

1.	Protect work, adjacent work, plants, shrubs and personal property at all times by suitable covering. Upon completion of the work, remove all paint and varnish spots from the floors, glass, hardware and other surfaces. Remove from the premises all rubbish and accumulated materials of whatever nature not caused by others and leave work in clean, orderly and acceptable condition. Touch up or re-paint all material damaged, marred, or otherwise not acceptable for final inspection.

D.	PAINTING SCHEDULE:

1.	Submit detailed list of paint materials to Project Manager, secure approval prior to purchase.  Paint list to show product name, vehicle, dry film thickness at suggested rate of coverage, percent of solids by weight and volume, re-coat drying time and intended method of application.

2	Contractor shall determine the location of all areas to receive painting and include in his base bid.

- UNIT PRICE BID FORM

Project:

- Bid Date:

- Bidder's Information:

Company Name:

- Address:

Telephone:

- FAX

Contact:

- Title:

- E-mail:			_______________________________________________

- The Undersigned

1.	Acknowledges the inspection and acceptance of:

A:	All bid documents as listed in the request for proposal;

B:	The Work Site, including areas available for material storage, handling and disposal, as well as access to subject roofs.

C:	Familiarity with applicable and controlling Federal, State, and Local Laws, Statutes, Codes, and Regulations

2.	Agrees

A:	To Hold this Bid open for a period of 60 days from the Bid Date;

B:	Upon written notice, to enter into and execute a Contract with the Owner based upon the Bid Documents as proposed with in seven (7) days;

C:	That the bidder has received the following Addenda:

No:                  	Date:                 		No:                  	Date:

No:                  	Date:                 		No:                  	Date:

No:                  	Date:                 		No:                  	Date:

- Bidder proposes to complete the work described in the scope of work and the construction documents for the sum of and itemized as follows.  Please note that unit prices are for use by Owner for the purpose of comparing proposals.  Unit Prices should be for actual conditions observed in the field by the contractor prior to submission of this bid. A Tally sheet per building itemizing the quantities for each face of the building will be prepared by the Owner and the Contractor prior to the execution of the contract.

4.	UNIT PRICES

Unit prices shall be all inclusive.  Labor  and material costs per unit shall include all costs for labor, equipment, supervision,  insurance, markups for profit and overhead, etc.  No additional markup shall be allowed over these costs.

| Work Item | Unit (SF, LF, Ea., etc) | Labor Cost/ Unit | Material Cost/Unit | Total Replacement Cost/Unit |
| --- | --- | --- | --- | --- |
| Metal Stairs – complete set Wood Stairs – complete set | ea |  |  |  |
| Stair pans and riser | ea |  |  |  |
| EIFS Repairs | SF |  |  |  |
| Trim material for use with full paint jobs ( RS Cedar or Pressure Treated) |  |  |  |  |
| 1 x 2 | Lineal foot |  |  |  |
| 1 x 4 | Lineal foot |  |  |  |
| 1 x 6 | Lineal foot |  |  |  |
| 1x 8 | Lineal foot |  |  |  |
| 1 x 10 | Lineal foot |  |  |  |
| 1 x 12 | Lineal foot |  |  |  |
| 2 x 2 | Lineal foot |  |  |  |
| 2 x 4 | Lineal foot |  |  |  |
| 2 x 6 | Lineal foot |  |  |  |
| 2 x 8 | Lineal foot |  |  |  |
| 2 x 10 | Lineal foot |  |  |  |
| 2 x 12 | Lineal foot |  |  |  |
| 4 x 4 | Lineal foot |  |  |  |
| 4 x 6 | Lineal foot |  |  |  |
| 4 x 8 | Lineal foot |  |  |  |
| 4 x 10 | Lineal foot |  |  |  |
| 4 x 12 | Lineal foot |  |  |  |
| 6 x 6 | Lineal foot |  |  |  |
| Sheathing |  |  |  |  |
| 1/2" CDX Plywood | Sheet |  |  |  |
| 3/4” CDX Plywood | Sheet |  |  |  |
| WP gypboard | Square foot |  |  |  |
| Hardi Panel | Sheet |  |  |  |
| T1-11 3/8" Plywood | Sheet |  |  |  |
| T1-11 5/8" Plywood | Sheet |  |  |  |
| Siding |  |  |  |  |
| Hardiplank |  |  |  |  |
| 6" Colonial Rough | Lineal foot |  |  |  |
| 8" Colonial Rough | Lineal foot |  |  |  |
| 12" Colonial Rough | Lineal foot |  |  |  |
| RS Cedar |  |  |  |  |
| 6" Beveled | Lineal foot |  |  |  |
| 8" Shiplap | Lineal foot |  |  |  |
| 8" Lap n Gap | Lineal foot |  |  |  |
| Cedar shingle | Square Foot |  |  |  |
| Hardboard |  |  |  |  |
| 8" Lap n Gap | Lineal foot |  |  |  |
| 8" Masonite | Lineal foot |  |  |  |
| 12" Masonite | Lineal foot |  |  |  |
| Shakertown shingle |  |  |  |  |
| 7" Colonial Even Butt | Lineal foot |  |  |  |
| 14" Colonial Even Butt | Lineal foot |  |  |  |
| Vinyl Siding Include “J’channel and access. |  |  |  |  |
| 5” reveal | Lineal foot |  |  |  |
| 8” reveal | Lineal foot |  |  |  |
| Structural members |  |  |  |  |
| 2"x4" wall stud | Each |  |  |  |
| 2"x6" wall stud | Each |  |  |  |
| 2"x4" sill | Lineal foot |  |  |  |
| 2"x6" sill | Lineal foot |  |  |  |
| Top plate on wall | Lineal foot |  |  |  |
| 2"x10" joist | Lineal foot |  |  |  |
| 3"x10" beam | Lineal foot |  |  |  |
| 4"x10" beam | Lineal foot |  |  |  |
| 4"x12" beam | Lineal foot |  |  |  |
| 4"x16" beam | Lineal foot |  |  |  |
| 6"x10" beam | Lineal foot |  |  |  |
| 6"x12" beam | Lineal foot |  |  |  |
| 6"x16" beam | Lineal foot |  |  |  |
| 8"x10" beam | Lineal foot |  |  |  |
| Window headers less than 42" ( includes sheet rock) | Each |  |  |  |
| Window headers greater than 42" but less than 96" (incl sheet rock) | Each |  |  |  |
| R-11 batt insulation 16"x8' | Each |  |  |  |
| Simpson HD-5 seismic hold down w/ 5/8" Hilti Epoxy anchor | Each |  |  |  |
| Fencing |  |  |  |  |
| 48" Fence (Metal) w/ 1/2" pickets 4"oc & 1" rails | Lineal foot |  |  |  |
| 48" Fence (Wood) w/ RS Cedar Pickets, 2x6 Top Cap & 1x4 band | Lineal foot |  |  |  |
| 72" Fence (Metal) w/ 3/4" pickets 6"oc & 1" rails | Lineal foot |  |  |  |
| 72" Fence (Wood) w/ RS Cedar Pickets, 8’ section w/post | Lineal foot |  |  |  |
| Steel posts for wood fence (Add) | Lineal foot |  |  |  |
| Pressure treated split rail fence | Lineal foot |  |  |  |
| Concrete repairs |  |  |  |  |
| 4" concrete w/ 3/8" RB @ 24" oc each way, R & R |  |  |  |  |
| 200-500 | Square feet |  |  |  |
| 501-1000 | Square feet |  |  |  |
| 1000 + | Square feet |  |  |  |
| 18" gutter & curb R & R | Lineal foot |  |  |  |
| 6" box curb, R & R w/ 1- 3/8" RB | Lineal foot |  |  |  |
| 42" wide sidewalk, accessible location | Square foot |  |  |  |
| 42" wide sidewalk, in-accessible location | Square foot |  |  |  |
| Concrete grind sidewalk offset | Each |  |  |  |
| Gutters and Downspouts |  |  |  |  |
| Seamless aluminum gutter trough 5” | Lineal foot |  |  |  |
| Downspouts (2"x3") | Lineal foot |  |  |  |
| Downspouts (3"x4") | Lineal foot |  |  |  |
| Concrete splashblocks | Each |  |  |  |
| Stucco Repairs |  |  |  |  |
| Stucco repairs | Square Foot |  |  |  |
| Water Proofing |  |  |  |  |
| Vulkem 350/351 | Lineal foot |  |  |  |
| Flashing | LF |  |  |  |
| Rail Road Ties 8x8x8 | Each |  |  |  |
| Brick rowlock repairs | LF |  |  |  |
| Painting and Full Caulking Unit Price: |  |  |  |  |
| 100% paintable surface | Per unit |  |  |  |
| 75% paintable surface | Per unit |  |  |  |
| 50% paintable surface | Per unit |  |  |  |
| 25% paintable surface | Per unit |  |  |  |
| Trim only | Per unit |  |  |  |
| Balconies only | Per unit |  |  |  |
| Second coat or full prime coat | Per unit |  |  |  |
|  | Per Unit |  |  |  |
|  | Per Unit |  |  |  |
|  | Per Unit |  |  |  |
|  | Per Unit |  |  |  |
|  | Per Unit |  |  |  |
| Mill Pond | Per Unit |  |  |  |
|  | Per Unit |  |  |  |
|  | Per Unit |  |  |  |

5.	Time of Completion

The Undersigned hereby affirms and states that if awarded the contract for said work, weather permitting, he will commence work within fifteen (15) days after the date of commencement as listed in the contract and will complete the work as follows:

working days

working days

working days

working days

working days

working days

working days

working days

Working days are considered to be Monday through Saturday, inclusive but exclude any federal or state holidays.

- By submitting this proposal the Undersigned agrees that if this proposal is accepted within the sixty (60) days after the Bid Date, or at any time prior to the Bid being withdrawn by way of written notification sent by Certified Mail, the Bidder is liable to the Owner for any damages the Owner may suffer by failure of the Bidder to enter into a Contract.

7.	Housing Stipulation:

Contractor shall specify if housing will be needed for their crews for the duration of this work.  All Housing cost are the sole responsibility of the Contractor.  In the event housing is required, the Owner can make arrangement for the contractor crews to occupy units on site or in neighboring properties at the discretion of the Owner.  The contract agrees to enter in a standard lease agreement for the housing provided including posting of all required security deposits.  The lease agreement will stipulate that the lease will be terminated upon completion of the work with no cancellation penalty. Contractor shall pay the cost of housing directly to the site on the first of ever month. Contractor is responsible for the cost of utilities, rental furnishings and other charges associated with the use of the rental premises.  Contractor is responsible for the cost of any damage caused to the premises during their occupancy, and Owner has the right to deduct the cost of any necessary repairs from the security deposit.

Housing required:			YES				NO

Number of housing units required

Type of unit requested 			2 BDRM			1 BDRM

In Submitting this bid, it is understood that the right is reserved by the Owner to reject any and all bids, and to select or reject any or all portions of Alternate bids for a period of Sixty (60) days from the Bid Date with out notification to the Bidders.

Company:                                        				 Date:

Signed:                              					Title:

Name:                               				  	Corporate Seal:

The Following is a sample of the building Talley sheet that will be required to be filled out per field conditions and before contract execution. Please not that each building Talley sheet will be specific for each property.

Building # – Talley Sheet

Note: Face building for FRONT, proceed Clockwise to RIGHT SIDE, REAR and LEFT SIDE

| Work Item | Front | Right side | Rear | Left side | Estimated Material total | Total Replacement Cost/Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 1x4 Trim |  |  |  |  |  |  |
| 1x6 – Trim |  |  |  |  |  |  |
| 1x8 – Trim |  |  |  |  |  |  |
| 1x10 – Trim |  |  |  |  |  |  |
| 1x12 – Trim |  |  |  |  |  |  |
| 2x2 – PT |  |  |  |  |  |  |
| 2x4 – PT |  |  |  |  |  |  |
| 2x6 – PT |  |  |  |  |  |  |
| 2x8 – PT |  |  |  |  |  |  |
| 2x10 – PT |  |  |  |  |  |  |
| 2x12 – PT |  |  |  |  |  |  |
| Stucco repairs |  |  |  |  |  |  |
| Painting |  |  |  |  |  |  |
| Shingle siding |  |  |  |  |  |  |
| Flashing |  |  |  |  |  |  |
| Gutter/Scuppers |  |  |  |  |  |  |
| Downspouts |  |  |  |  |  |  |
| Caulking |  |  |  |  |  |  |