ASBESTOS OPERATIONS, MAINTENANCE and REPAIR ACTION PLAN

January 5, 2017

- INTRODUCTION

1.1 Objectives

The principal objective of this Operations, Maintenance, and Repair Action Plan ("O&M" Plan)is to minimize the airborne asbestos exposure of all building occupants by reducing the potential for uncontrolled and/or unintentional disturbance of or damage to asbestos-containing building materials (ACBM) and presumed asbestos-containing building materials (PACM). The degree to which the various provisions come into play will be determined largely by the degree of activity potentially affecting ACBM and PACM (e.g., renovation activity).

The O&M Plan is prepared in such a way that it may be easily referred to and applied to various levels of activity and can be revised as required, and is intended for use by program administrators and maintenance personnel.

Asbestos-containing building material which has maintained its original cohesive and adhesive integrity, and which remains undisturbed, presents minimal airborne asbestos exposure hazard. Damaged, deteriorating, or disturbed ACBM and PACM may release asbestos fibers into the air creating a potential health hazard. Non-friable ACBM and PACM will not typically release measurable quantities of fibers until being abraded, drilled, sanded, or otherwise becoming friable.

1.2 Policy Statement

This O&M Plan is intended to be a working document which will serve as a guide to program administrators and maintenance personnel in minimizing the risk of exposure to asbestos fibers to occupants, employees, visitors, vendors, and contractors.

Building Owner . recognizes the potential health hazards associated with asbestos fibers and has implemented this O&M Plan for the suspect buildings located on its properties which were constructed prior to 1981 and which therefor may contain asbestos in one or more of the building materials present in them.

This Program sets forth the recommended procedures for dealing with the suspect ACM present in these buildings and has been reviewed by and represents the policy of the Building Owner. The intention of the Building Owner in setting forth these procedures is to provide a safe and healthful living and working environment at their residential properties to the best of their ability.

1.3 Asbestos - General Information

Asbestos is the generic term for a group of naturally occurring fibrous minerals with high tensile strength, flexibility, and resistance to thermal, chemical, and electrical conditions.

Asbestos fibers enter the body through inhalation or ingestion and become embedded in the tissues of the respiratory or digestive systems. The symptoms of these diseases generally do not appear for 20 or more years after initial exposure.

Risk for exposure to airborne asbestos fibers is particularly high in the following workplaces:

1. Primary mining and manufacturing of asbestos products;

2. Secondary Manufacturing of asbestos-containing products, such as fire-resistant clothing, fire-resistant wall board, roofing materials, brake or clutch parts, and asbestos cement pipe;

3. Construction activities that include installation of asbestos-containing materials ACMs and/or PACM the demolition, removal, or disturbance or alteration of asbestos containing building materials, and;

4. Automobile brake and clutch repair activities.

Workers in the above listed categories and those workers who are in the area where such activities are being conducted are susceptible to exposure.

In addition, custodial workers or other employees who perform housekeeping, cleaning of waste and debris, and accompanying dust in an area containing accessible thermal system insulation or surfacing material or visible deteriorated ACM may be exposed above the permissible exposure level (PEL). The particular requirements that apply depend on the nature and extent of the work, on the materials involved, and on the results of exposure assessment.

At the federal level, the OSHA standard 29 CFR 1926. 1101 requires building owners to presume all suspect asbestos-containing building materials (ACBM and PACM), including thermal system insulation, surfacing materials, and vinyl and asphalt flooring materials, in buildings constructed prior to 1981 contain asbestos unless the material has been sufficiently sampled to prove the contrary.  California OSHA ("CalOSHA") standards at 8 CCR section 5208 are essentially the same.

In addition, the OSHA standards require building owners to take steps to identify ACM in their buildings, to keep records about the presence of ACMs, to post signs identifying areas with asbestos hazards, and to notify employees, tenants, contractors, and other employers of the presence of asbestos hazards who may be occupationally exposed to asbestos.

1.4 Sample List of Suspect Asbestos-Containing Materials

The following is a sample list of ACMs listed in the USEPA, 1990, Environmental Protection Agency, "Managing Asbestos In Place, A Building Owner's Guide to Operations and Maintenance Programs for Asbestos-Containing Materials", (Green Book) Appendix G:

Cement Pipes Elevator Brake Shoes

Cement Wallboard HVAC Duct Insulation

Cement Siding Boiler Insulation

Asphalt Floor Tile Breeching Insulation

Vinyl Floor Tile Ductwork Flexible Fabric Connection

Vinyl Floor Sheeting Cooling Towers

Flooring Backing Pipe Insulation

Construction Mastics Heating and Electrical Ducts

Acoustical Plaster Electrical Panel Partitions

Decorative Plaster Electrical Cloth

Textured Paints/Coatings Electric Wiring Insulation

Ceiling Tiles and Lay-in Panels Chalkboards

Spray-Applied Insulation Roofing Shingles

Blown-in Insulation Roofing Felt

Fireproofing Materials Base Flashing

Taping Compounds (Thermal) Thermal Paper Products

Packing Materials (for wall/floor penetrations) Fire Doors

High Temperature Gaskets Caulking/Putties

Laboratory Hoods/Table Tops Adhesives

Laboratory Gloves Wallboard

Fire Blankets Joint Compounds

Fire Curtains Vinyl Wall Coverings

Elevator Equipment Panels Spackling Compounds

Note: This list does not include every product material that may contain asbestos. It is intended as a general guide to show which types of materials may contain asbestos.

1.5 Definition of Terms

"Abatement" means procedures to control fiber release from asbestos-containing materials. This includes removal, encapsulation, enclosure, and repair.

"Amended water" means water to which a surfactant has been added to improve penetration and reduce fiber release.

"Asbestos" includes chrysotile, amosite, crocidolite, tremolite asbestos, anthophyllite asbestos, actinolite asbestos, and any of these minerals that has been chemically treated and/or altered. For purposes of 29CFR 1926. 1101 and this O&M Program, "asbestos" includes Presumed Asbestos- Containing Materials, as defined below.

"Asbestos-Containing Material" (as defined by 29CFR 1926. 1101 (Federal OSHA)) means any material containing more than one percent asbestos.

"Asbestos-Containing Construction Material" (as defined by 8CCR 1529 (California OSHA)) means any material containing more than one-tenth of one percent asbestos.

"Asbestos Program Manager" means an individual designated as the Building Management's representative and responsible for the building's operations and maintenance plan.

11 ASTM" means the American Society for Testing and Materials, 1916 Race Street, Philadelphia, PA 19103.

"Authorized person" means any person authorized by the employer and required by work duties to be present in regulated areas.

"Building Owner" means the subsidiary or affiliate of Essex Property Trust, Inc. which holds title to the property on which buildings containing ACM, suspect ACM or PACM are believed to be present.

"Certified Industrial Hygienist (C.I.H.)" means an industrial hygienist certified by the American Board of Industrial Hygiene.

"Class I Asbestos Work" means removal (abatement) of thermal system insulation (TSI) and surfacing material.

"Class II Asbestos Work" means removal of all other asbestos-containing materials not identified in Class I (floors, roofs, transite, gaskets, etc.).

"Class III Asbestos Work" means maintenance and repair operations disturbing asbestos containing materials. Requires a work plan.

"Class IV Asbestos Work" means housekeeping and custodial operations (including construction site cleanup).

"Competent person" means one who is capable of identifying existing asbestos hazards in the work-place and who has the authority to take prompt corrective measures to eliminate them, as specified in 29 CFR 1926.32(f). The duties of the competent person include at least the following: establishing the negative-pressure enclosure, ensuring its integrity, and controlling entry to and exit from the enclosures; supervising any worker exposure monitoring required by the standard; ensuring that all workers working within such an enclosure wear the appropriate personal protective equipment, are trained in the use of appropriate methods of exposure control, and use the hygiene facilities and decontamination procedures specified in the standard; and ensuring that engineering controls in use are in proper operating condition and are functioning properly.

"Decontamination Unit" means an enclosed area consisting of an equipment room, shower area, and clean room, which is used for the decontamination of workers, materials, and equipment contaminated with asbestos.

"Demolition" means the wrecking or taking out of any load-supporting structural member and any related razing, removing, or stripping of asbestos products.

"Encapsulant (sealant)" means a liquid material which can be applied to asbestos-containing material and which temporarily controls the possible release of asbestos fibers from the material either by creating a membrane over the surface (bridging encapsulant) or by penetrating into the material and binding its components together (penetrating encapsulant).

"Encapsulation" means the coating or spraying of asbestos material with a sealant.

"Encasement" means the covering of asbestos material with a hard-setting sealing agent.

"Enclosure" means the construction of air tight walls and ceilings between the asbestos material and the building environment, or around surfaces coated with asbestos materials, or any other appropriate scientific procedure as determined which prevents the release of asbestos materials.

"Employee exposure" means that exposure to airborne asbestos that would occur if the employee were not using respiratory protective equipment.

"EPA" means the Environmental Protection Agency, 401 M Street, S. W., Washington, DC 20460.

"Fiber" means a particulate form of asbestos, tremolite, anthophyllite, or actinolite, 5 micrometer or longer, with a length-to-diameter ratio of at least 3 to 1.

"HEPA-vacuum" means vacuuming equipment with a high-efficiency particulate air filter system, or to use such equipment.

High-efficiency particulate air (HEPA) filter" means a filter capable of trapping and retaining at least 99.97 percent of all monodispersed particles of 0.3 micrometers in diameter or larger.

"Management Plan" means the inspection report, laboratory (lab) analyses and response actions to be taken when asbestos-containing material (ACM and PACM) is in the building.

"Negative Air Pressure Equipment" means a portable local exhaust system equipped with HEPA-filtration. The system should be capable of maintaining a constant, low velocity airflow of filtered air from contaminated areas into adjacent uncontaminated areas, creating a negative pressure differential between the outside and inside of the Work Area. This negative pressure differential assures that air from the containment area will not migrate to uncontaminated areas.

"Negative Initial Exposure Assessment" means a demonstration by the employer, which complies with the criteria in 29CFR 1926. 1101 (f)(2)(iii), that employee exposure during an operation is expected to be consistently below the PELs.

"NESHAP" means the National Emission Standards for Hazardous Air Pollutants (40 CFR Part 6 1).

"NIOSH" means the National Institute for Occupational Safety and Health CDC - NIOSH, Building J N.E., Room 3007, Atlanta, GA 30333.

"Non-friable" means materials in a building which when dry may not be crumbled, pulverized, or reduced to powder by hand pressure.

"OSHA" means the Occupational Safety and Health Administration, 200 Constitution Avenue, Washington, DC 20210.

"Permissible exposure limit" means an airborne concentration of asbestos in excess of 0. 1 fiber per milliliter of air as an eight (8)-hour time-weighted average (TWA).

"Presumed Asbestos Containing Material" means thermal system insulation and subsurfacing material found in buildings constructed no later than 1980.

"Regulated area" means: an area established by the employer to demarcate areas where Class I, II, and III asbestos work is conducted, and in any adjoining area where debris and waste and such asbestos work accumulate; and a Work Area within which airborne concentrations of asbestos, exceed, or there is a reasonable possibility they may exceed the permissible exposure limit. Requirements for Regulated Areas are set out in 29 CFR 1926. 1101 (e)(6)..

"Removal" means the taking out or stripping of asbestos or materials containing asbestos.

"Renovation" means the modifying of any existing structure, or portion thereof, where exposure to airborne asbestos may result.

"Repair" means overhauling, rebuilding, reconstruction, or reconditioning of structures or substrates where asbestos present.

"Strip" means to remove friable or non-friable asbestos materials from any part of the building.

"Surfacing Material" means any friable asbestos-containing or presumed asbestos-containing material that is sprayed-on, trawled, or coated over a surface (e.g. acoustical ceiling material)

"Surfactant" means a chemical wetting agent that when added to water will improve penetration and reduce fiber release.

"Thermal System Insulation" (TSI) means any friable asbestos-containing or presumed asbestos-containing insulation material found on plumbing, processing, and HVAC systems.

"Visible emissions" means any emissions containing particulate asbestos material that are visually detectable without the aid of instruments.

"Wet cleaning" means the process of eliminating asbestos contamination from building surfaces and objects by using cloths, mops, or other cleaning tools which have been dampened with water, and by afterward disposing of these cleaning tools as asbestos contaminated waste.

2.0 DELINEATION OF RESPONSIBILITIES

The prevention of asbestos fiber exposure to building occupants requires commitment and proper implementation of the O & M Plan. Thus, the success of this Plan relies on the joint efforts and responsibilities shared by everyone involved: the Building Owner's Representative (Resident Manager) and/or Program manager, tenants, outside contractors and maintenance personnel. One persons improper actions could result in an asbestos fiber exposure affecting many or all personnel within the building, in addition to asbestos contamination of the outside environment. All personnel should realize the seriousness and hazards associated with airborne fibers and make their best effort to control or prevent its occurrence.

2.1.1 THE BUILDING OWNER'S REPRESENTATIVE (hereby referred to as the “Asbestos Program Manager”)

The community manager is the Asbestos Program Manager and is responsible for implementing and overseeing the O&M Plan.  . BENCHMARK ENVIRONMENTAL ENGINEERING (“BENCHMARK”) will assist the Asbestos Program Manager in the day-to-day maintenance management of the O&M Plan.

2.1.2 TRAINING OF THE ASBESTOS PROGRAM MANAGER

The Asbestos Program Manager has received adequate training to perform the assigned duties in accordance with the engineering controls and work practices required for Class III asbestos work. The training should include the following Cal/OSHA title 8 CCR 1529 Asbestos Class III Awareness Course (8 -hours).

2.1.3 The Asbestos Program Manager's responsibilities include, but are not limited to the following:

1. Overseeing all asbestos-related activities in the building and Oversee all O&M related work, which conforms to work practices and engineering controls for Class III asbestos work.

2. Reviewing this O&M Program annually or semi-annually to determine whether it should be updated.

3. Maintenance of all record-keeping documentation of records and forms, such as training.

4. Ensure that:

- Work Permit System is initiated and complied with that may disturb asbestos.

- Notification and Warning Program is initiated and continued,

- The Monitoring Program is initiated and continued,

- Management, maintenance and custodial staff is properly trained,

- Affected workers are subject to a surveillance program, and that they follow proper hygiene practices,

- Maintenance personnel are complying with procedures Identified in the O&M Program,

- Emergency procedures are complied with,

- No new ACM is installed in the building (Section 17), and

- Records of asbestos-related activities are kept properly (Section 18).

The Asbestos Program Manager may delegate these tasks to others that have completed Cal/OSHA title 8 CCR 1529. Asbestos Class III Awareness Course (8-hours). However, the asbestos manager should maintain responsibility for overall compliance with this Operations, Maintenance, and Repair Plan.

2.2 ASBESTOS PROGRAM MANAGER (Class III O & M Workers )

The O&M Plan is designed for trained and qualified maintenance personnel to conduct limited Class III and Class IV Asbestos Work operations in accordance with procedures outlined within the O&M, and Repair Action Plan, and Standard Operating Procedure (SOP). All other asbestos related work will be contracted to one of the asbestos contractor on-call and under a master service agreement with Essex Property Trust, Inc. and shall be overseen by Essex Property Trust, Inc.

2.3 MAINTENANCE PERSONNEL

2.3.1 Training of the Maintenance Personnel

Maintenance personnel who will be conducting limited Class III and Class IV asbestos operations shall have successfully completed the company's asbestos awareness training program. This course adopts the material set out in CCR Title 8, section 1529(k)(9). Such course shall include available information concerning the locations of PACM and ACM, and instruction on recognition of damage, deterioration, and delamination of ACBM and PACMs. In addition, a hands on portion of the training is required. Such course should take at least seven hours (7). In addition, all maintenance personnel must comply with the repair and cleaning procedures specified in this Plan.

New members of the maintenance staff will attend a (2) hour Class IV Asbestos Awareness Course Cal/OSHA 1529(K)(9)(H) during the workers initial orientation. The training program shall be conducted in a manner that the employee is able to understand. The employee will need to be informed of the following: History of Asbestos, Uses of Asbestos, the types and locations of ACBM and PACM, Health Effects Associated with Asbestos, Respiratory Protection. In addition, the worker should be trained following the above mentioned guideline prior to performing any asbestos related class IV work.

2.3.2 Duties of the Maintenance Staff

For purposes of this Asbestos Operations, Maintenance, and Repair Action Plan, maintenance staff shall be defined as personnel responsible for the operations and maintenance of the building (O&M). Maintenance personnel are required to comply with all rules outlined below:

- Maintenance staff personnel may engage in repair and maintenance operations, where "ACM and PACM", including TSI and surfacing ACM and PACM needs to be disturbed.

- So long as the maintenance person complies with all sections of CCR Title 8, Section 1529(g)(9) and all other applicable sections in order to minimize the exposure to the employee performing the asbestos work and to bystander employees.

- Maintenance staff shall comply with these requirements, in addition to any other directions given by the Asbestos Program Manager that affect live safety.

- Immediately report any evidence of disturbance or damage of ACM or PACM to the Asbestos Program Manager, such as any unauthorized activities involving suspect ACM and PACM, any and all spills or other releases of known ACM or PACM, and the presence of any debris suspected to contain asbestos so that the property management can respond as appropriate. If the Asbestos Program Manager is unavailable, contact the Regional Portfolio Manager.

- Maintenance personnel shall not store any items within five (5) feet of any white, asbestos containing insulated pipe which is not enclosed inside a false column.

- All maintenance personnel shall take precautions to store items far enough away from the non-enclosed white asbestos containing insulated pipe such that the stored items will not touch the exposed insulated pipe and potential debris fall-out from the pipe will not fall onto the stored items.

- Maintenance personnel shall observe and comply with any signage posted within the building pertaining to asbestos warnings, notifications or restricted areas.

- Do not dry sweep or dust any debris suspected to contain asbestos. If possible, wet down the material and restrict access to or isolate the area.

- Comply with all Health and Safety Procedures established under Awareness Training.

- Participate in annual (2) hour O & M Personnel Training course work, including information covered in the Awareness Training course, along with training on Protective Clothing and Equipment, Summary of Regulations, Emergency Response Actions and Contracts, Standard Operating Procedures for Maintenance.

- An unplanned ACM and PACM emergency may be the result of earthquake, fire, water leaks, direct or indirect physical disturbance, or other unforeseen incidences and activities causing (or likely to cause) increased levels of airborne asbestos fibers within the building.

- All spills and sudden releases or potential releases of asbestos shall be contained and cleaned up as soon as possible under the direction of the Program Manager.

The primary goal to achieve after an ACM or PACM emergency is to prevent the impending or further asbestos contamination of building air space.

Maintenance Staff personnel shall follow these procedures in the event of an asbestos incident or emergency:

- All contaminated or potentially contaminated areas shall be isolated and access restricted to the extent feasible.

- Post asbestos warning signs and notify the Asbestos Program Manager.

- Appropriately adjust and/or isolate the ventilation system in the contaminated area.

- Access to the contaminated area shall be controlled and authorized only to personnel who are properly trained and protected with appropriate clothing.

- Personnel working in the contaminated area should wear personal air monitors as appropriate for their specific work duties inside the work area. Perform work as directed by the Asbestos Program Manager.

BENCHMARK shall determine whether there is a potential for asbestos exposure in the contaminated area to reach or exceed the "action level", 0.1 fibers per cubic centimeter (f/cc) 1 of air. Air monitoring data shall be obtained as soon as physically possible to verify the anticipated airborne asbestos levels.

Emergency response actions to ACM and/or PACM emergencies in excess of the action level (as determined by Benchmark) shall be performed by outside abatement contractor services. Emergency response actions in contaminated areas that are not anticipated to exceed the action level and anticipated to fall below airborne asbestos exposure levels of 0.01 F/cc may be performed by properly trained, protected and equipped personnel.

2.4 SERVICE CONTRACTORS AND VENDORS

2.4.1 RULES FOR CONTRACTORS

For purposes of this Asbestos Management plan, the term "Contractors" is defined as any person contracted to provide or perform services on behalf of a tenant. The term contractors includes but is not limited to personnel such as, general contractors, subcontractors, telephone installers, utility workers, etc. Contracts with service trades and vendors should include the following provisions to ensure that the Contractor's workers can and will follow appropriate work practices:

1. Contractors and Vendors will be notified of the types and locations of ACBM and PACM at the sites where work is to be performed. Each service Contractor shall be responsible for informing their employees of the presence of ACBM, PACM and potential health hazards associated with asbestos.

2. Employees of Contractors and Vendors that intend to perform work on any ACBM and PACM(s) or suspect ACBM will be properly trained in performing asbestos related activities prior to performing any asbestos-related work on Essex Property Trust, Inc. properties.

3. Assurance that the Contractor's workers will abide by all provisions of this O&M Plan and Master Asbestos Abatement Specifications.

4. Written work practices must be submitted by the Contractor for approval or Modification by the Asbestos Program Manager when ACBM and PACM are to be impacted By the Contractor.

5. Agreement to abide by the work practices as accepted by the Asbestos Program Manager,

6. Copies of respiratory protection, medical surveillance, and worker training documentation as required by OSHA, EPA, and/or state regulatory agencies from workers who may be exposed to asbestos hazards.

7. Any other information deemed necessary by the building management's legal Counsel.

If the Contractor employee should request or require entry into an area where friable damaged asbestos-containing material is located and to which access is restricted, the Asbestos Program Manager shall obtain copies of respiratory protection, medical surveillance, and worker training documentation. The Asbestos Program Manager shall also secure the signature of the worker on a comprehensive release form, which states the following:

1. That the worker has been informed that the designated area contains friable damaged asbestos-containing material,

2. That access to the area is restricted to persons with asbestos training who are wearing respiratory protection at all times,

3. That the worker understands that asbestos is a recognized health hazard and that asbestos fibers can cause lung disease and cancer,

4. That the worker assumes full responsibility for his/her own protection and Welfare when entering the restricted area and that he/she will hold the building management and its agents harmless from any injury claim related to asbestos exposure, and

5. The name of the worker, the company employing him/her, the nature of the work, and the date.

2.5 RULES FOR TENANTS

For the purposes of this O&M Plan "Tenant" shall refer to any individual or entity that leases space in the building and its agents, employees, contractors and invitees. Tenants include owners, lessees, subleasees, or agents of the owners of any other building or part of a building located on the property, including, but not limited to, the state or another public entity. Each tenant in the building, its employers, or any person (including a Contractor) acting on a tenant's behalf, is required to comply with the following rules in meeting joint responsibility to prevent asbestos fiber release and exposure in the building.

- Under no circumstances shall a tenant, or any person acting on behalf of such tenant, engage in work involving the disturbance of any building materials, including the removal of any fixture from floors, walls or ceilings, or any work above the suspended ceiling, without prior written permission of the Asbestos Program Manager.

- Under no circumstances shall the tenant or any person acting on behalf of such tenant to undertake any construction, renovation, maintenance, repair, alteration, addition, modification, improvement or like work in or to the premises leased without the prior written permission of the Asbestos Program Manager.

- All tenants shall immediately report any evidence of disturbance or damage of ACM and PACM to the Asbestos Program Manager. Any unauthorized activities involving suspect ACM, and all spills or other releases of known ACM and PACM, and the presence of any debris suspected to contain asbestos so the Asbestos Program Manager or training staff can respond as appropriate. If the Resident Manager is unavailable, contact the Regional Portfolio Manager

- Neither tenant nor any person acting on such tenant's behalf, shall touch or disturb the ACM and/or PACM identified in the building, including any debris suspected to contain asbestos, unless specifically authorized in writing by the Asbestos Program Manager on a case-by case basis

- The tenant shall not store any items within five (5) feet of any white, asbestos containing insulated pipe which is not enclosed inside a false column, and shall take precautions to store items far enough away from the non-enclosed white asbestos containing insulated pipe such that the stored items will not touch the exposed insulated pipe and potential debris fall-out from the pipe will not fall onto the stored items.

- Tenants shall observe and comply with any signage posted within the building pertaining to asbestos warnings, notifications or restricted areas.

- Notify the Asbestos Program Manager before conducting planned renovations of any size to allow the Asbestos Program Manager to review the renovation plans and help prevent building tenants from unknowingly disturbing ACBM and PACM.

All tenants (including persons acting on their behalf) shall comply with these requirements, in addition to any other directions given by the Asbestos Program Manager.

Tenants shall further follow these instructions for asbestos incidents and emergencies:

- Immediately notify any other personnel working in the area, including supervisors, of the incident and its asbestos-related nature.

- Do not delay notification, even if the asbestos incident seems apparent to all personnel in the area.

- All Tenant personnel should rapidly evacuate the area, closing access doors upon exit (if appropriate under the circumstances).

- Immediately contact the Asbestos Program Manager and inform this person of any observed or suspected asbestos incident, it nature, its location, and other facts as appropriate.

The assistance of building tenants should also be required in completing the provisions of the Notification Program.

2.6 THE OPERATIONS, MAINTENANCE AND REPAIR PROGRAM

BENCHMARK, shall provide asbestos management services and shall act in an advisory role to Building Owner for asbestos management issues.

BENCHMARK services shall include:

- Being available by telephone at all times for asbestos related emergencies

- Monitoring of the O&M Plan to maintain compliance with the latest local, state and federal asbestos related regulations.

- Design and develop asbestos work plans, drawings and specifications for emergency and abatement activities.

- Project management and air monitoring for asbestos work activities by outside abatement contractors.

- Periodic air sampling to document asbestos fiber levels on in various properties.

- Providing asbestos training Class III and Class IV.

- Reporting to the Building Owner's senior staff regarding any and all asbestos fiber release episodes observed or suspected, and

- Assisting in arranging an appropriate response action.

This document does not and is not intended to cover each and every asbestos management responsibility of the parties referenced herein, but is intended to serve as a guide to those parties.

3.0 INVENTORY OF ASBESTOS-CONTAINING MATERIALS PRESENT

3.1 Presumed Asbestos-Containing Materials (PACM)

The following list of materials is to be considered PACM(s) at all Building Owner properties (built pre-1981) encompassed by this O&M Plan:

1. Ceiling material (sprayed-on or suspended ceiling tile);

2. Various types of Resilient Flooring (Sheet and Tile) and associated construction mastics;

3. Cloth tape on HVAC ducting seams;

4. Pipe insulation (lagging and mudded joints);

5. Gypsum wallboard (sheet rock) and taping compound, and;

6. Base coving (baseboard) and associated mastic.

7. Exterior stucco;

8. HVAC joint tape;

9. Roofing material.

Work, which may or will damage or disturb Presumed ACM should proceed exactly as if the material was known to be ACM. Alternatively, Presumed ACM may be sampled and analyzed by polarized light microscopy (PLM) to determine if the material is indeed ACM prior to such work. A minimum of three representative samples per homogeneous material should be collected and analyzed to determine a material to be non-ACM.

4.0 ASBESTOS RULES AND RESPONSIBILITES

4.1 INSTRUCTION FOR NOTIFICATIONS

Under the California Health and Safety Code, Section 25915, chapter 10.4 "Asbestos Notification," the Building Owner is required to provide written notification to all building employees (including custodial and operations and maintenance personnel), tenants (including lessees and subleases), and Contractors (including employees of Contractors and subcontractors), informing them of the following information with respect to commercial buildings constructed prior to 1979 where asbestos containing materials are known to be present:

- The location of ACM and/or PACM in the building, including the content and type of asbestos contained in the materials and the locations of the ACM and/or PACM

- The potential health risks or impacts that may result from exposure and inhalation to airborne asbestos fibers to the extent known to Building Owner

- Notice that disturbance of ACM and/or PACM (such as moving, drilling, boring, sanding, etc.) by personnel untrained in asbestos handling should not be attempted because of the potential health risk resulting from exposure to airborne asbestos fibers.

Regulation 29 CFR 1926. 1101 requires building and facility owners, notify in writing, or by personal communication between the owner and the person to whom notification must be given or their authorized representatives. Notification must be given to the following:

1. Prospective employers applying or bidding for work whose employees reasonably can be expected to work in or adjacent to areas containing ACMs;

2. Employees of the Owner who will work in or adjacent to areas containing ACMs;

3. On multi-employer work sites, all employers of employees who will be performing work within or adjacent to areas containing ACMs

4. Tenants who will occupy areas containing ACMs.

4.1.2 Notification of Tenants

All new tenants and building occupants should be provided with information to the extent in the possession of the Building Owner (e.g. through the distribution of printed notices or through informational seminars) which addresses the following:

1. The locations, types, and conditions of known or presumed ACBM and PACMs in buildings at the complex (refer to section 3.0),

2. The fact that the mere presence of ACBM and PACM does not represent a health hazard and according to EPA guidance, removal of asbestos is often not the best option, improper removal can create a hazard where none existed previously,

3. That Asbestos presents a hazard when asbestos fibers become airborne and are inhaled,

4. The response actions which building management has taken including the development and implementation of the O&M Plan, Maintenance and custodial personnel are trained to avoid disturbing ACBM and PACM,

5. All ACBM and PACM is inspected periodically and additional measures will be taken if necessary to protect the health of building occupants,

6. Do not disturb known or presumed ACBM and PACM,

7. Report any evidence of disturbance or damage to ACBM AND PACM to the Manager,

8. Report any dust or debris that might come from the ACM or presumed ACM.

9. Report any change in condition to the ACM or presumed ACM.

Special provisions may need to be arranged if some building occupants do not speak English or are illiterate.

4.1.3 Abatement Notification

Tenants, employees, and all subcontracted employees at the scheduled site will be notified in writing prior to abatement and notified upon completion of the abatement project. Notifications will include the following,

1. Material scheduled for abatement and location,

2. Time schedule for abatement project,

3. Clearance results upon abatement completion.

4.2 ASBESTOS WARNING SIGNS

Warning labels will be permanently affixed immediately adjacent to any friable or non-friable asbestos-containing material located in routine maintenance or service areas not generally accessible to the public.

ASBESTOS WARNING SIGNS (Posted in routine maintenance areas, such as telephone and electrical closets, mechanical rooms, etc.)

CAUTION:

ASBESTOS -  CANCER AND LUNG DISEASE HAZARD

DO NOT DISTURB WITHOUT PROPER TRAINING AND EQUIPMENT

CONTACT BUILDING MANAGER AT OFFICE BEFORE PERFORMING ANY WORK

RESTRICTED ASBESTOS WORK AREA (Posted in work areas where asbestos materials are being disturbed for maintenance repair or removal activities.)

DANGER:

ASBESTOS - CANCER AND LUNG DISEASE HAZARD

AUTHORIZED PERSONNEL ONLY KEEP OUT

RESPIRATORS AND PROTECTIVE CLOTHING

ARE REQUIRED IN THIS AREA

CONTACT THE BUILDING MANAGER AT OFFICE  IF ACCESS IS REQUIRED

5.0 SERVICE REQUEST PERMIT SYSTEM

Initiating a work permit system is an established method to help control maintenance, and renovation activities, which could result in the disturbance of asbestos-containing building materials (ACBM and PACM). Under this permit system, all requests for maintenance or renovation activities must be reviewed by the Asbestos Program Manager prior to the issuance of a work order (i.e. the Asbestos Program Manager must be aware of ACM and PACM locations, types and if material will be impacted).

Persons requesting work must submit a Request/Authorization Form, which gives the location, date, and nature of the proposed work. Upon receipt of a maintenance call from a resident, the Asbestos Program Manager should complete a work Request/Authorization Form and the following steps should be taken:

1. The Asbestos Program Manager should refer to the suspect ACM(s) in section 3.0 of the O&M Plan to determine if ACM is present in the area of the proposed work.

2. If suspect ACM is not present in the area of the proposed work, the Asbestos Program Manager may authorize the work to proceed by checking the box indicating suspect ACM is not present in area of the proposed work, and sign his/her name.

3. If ACM or Suspect ACM is present in the area of the proposed work, the Asbestos Program Manager should categorize the proposed work as one of the following:

a. Work which is unlikely to cause disturbance of ACM,

b. Work which may cause disturbance of ACM, or PACM,

c. Work which will cause disturbance of ACM or PACM.

4. If the proposed work is unlikely to cause disturbance of suspect ACM, the Asbestos Program Manager may authorize the work to proceed by :

- checking the box Indicating ACM is present but unlikely to be disturbed,

- indicating the type and location of ACM present, and

- signing his/her name to the Work

- completing a Request/Authorization Form.

5. If the proposed work may cause disturbance of suspect ACM, the Asbestos Program Manager may authorize the work to proceed and indicate that suspect ACM is present and may be disturbed, indicate the type and location of suspect ACM present, direct that worker must take special precautions to avoid disturbing the ACM, sign his/her name to the Work Order/Authorization Form.

6. If the proposed work will cause a disturbance to ACM or PACM, the Asbestos Program Manager must follow the following criteria prior to authorizing the Work Order:

- If the proposed Work Order will disturb any ACM or PACM which will be classified as Class III asbestos work, the Work Order must be denied and routed to the Asbestos Program Manager.

- If the work will only disturb gypsum wallboard and joint/taping compound, the Work Order can be authorized by the Asbestos Program Manager only if the quantity of wallboard material meets the criteria of small scale short duration.

- If the Work orders meet the criteria for small scale short duration, the Asbestos Program Manager must instruct the Maintenance personnel to follow the Standard Operating Procedure for Class III work practices and engineering controls prior to signing his/her name to the Work Order/Authorization Form.

5.1 SMALL-SCALE ASBESTOS PROCEDURES

Cal/OSHA defines "Small-Scale, Short Duration Activities" as operations for which a negative pressure enclosure is infeasible, impractical, or unsafe due to the small scale of the task. Examples of these tasks include, but are not limited to:

- Attaching devices to ACM walls

- Removing devices from ACM walls

- Drilling holes in ACM walls, floors, ceilings

- Repairing small holes in drywall

- Removing small portions of damaged drywall

- Maintenance of light fixtures on sprayed-on or troweled-on acoustical ceilings and other surfaces

- Removing suspended ceiling tiles for access to plenum or crawl spaces

- Repair or replace vinyl floor tile

- Disturbing tape on piping or duct work

- Repairing damaged piping insulation

- Removing cobwebs from sprayed-on ceilings

- Removing carpet from existing vinyl tile floors

- Removing baseboard molding and mastic

- Working in close proximity to TSI and/or sprayed-on or troweled-on surface material

Necessary precautions If damage to surfaces is unavoidable

Several controls and work practices, used singly or in combination, need to be employed to effectively reduce asbestos exposures during small Class III repair and maintenance operations. These include wet methods (which must be used whenever practicable).

5.2 WORK PRACTICES FOR SMALL-SCALE ACM PROCEDURES

In the event that the use of respirators is not a part of the normal operation policy of the company, strict attention must be given to the methods used in the following maintenance scenarios. It must be understood that the use of a respirator is not a choice made without a backup program which involves training, medical clearance, individual fit testing, documentation and record keeping.

The use of respirators and other protective equipment is triggered when the workman’s activity creates an environment that exceeds the OSHA action levels which are 0.1 f/cc for an 8-hour time weighted average or l.0 f/cc for a 30-minute sampling period (referred to as the excursion limit). Simply stated, you cannot disturb an asbestos containing surface in a way that will cause dust. There are several ways that this can be accomplished. By adhering to each or a combination of these control methods, certain maintenance tasks can be accomplished without creating dust.

The following control methods should be used in dealing with or working in areas where ACM or PACM is present:

- First and most important control method is to keep the task that could disturb ACM to a small enough size so dust emissions can be adequately controlled.

- Second, the use of water or wet methods that will act as a dust deterrent. This method must be limited to areas that are not electrical in nature.

- Thirdly, the use of a HEPA filtered vacuum used in conjunction with wet methods to eliminate particles before they can be dispersed.

5.3 Maintenance staff performing the work must be equipped at the minimum with the following:

1.    HEPA vacuum cleaner to remove debris on the floor.

2.    Duct tape and a 10' x 10' 6-mil piece of polyethylene sheeting to protect the ground landscape and any equipment immediately adjacent to the work area.

3.    Construction tape and warning signs to discourage intruders near the area.

4.    Six-mil polyethylene labeled bags for asbestos containing waste.

5.    Spray bottle, water and paper towels.

5.4 The maintenance staff performing the work must follow the ensuing procedures:

- Whenever possible, make use of existing doors to barricade the work location. If doors cannot barricade the area, use construction warning tape and signs for at least 20 feet in all directions.

- Remove any equipment, furniture or any movable objects away from directly below the spot where sprayed-on fireproofing material has to be disturbed or removed.

- Tape a piece of 10'x10' 6 mil polyethylene sheeting below the spot where the material has to be disturbed. Cover all surfaces.

- Using amended water in a Hudson sprayer, lightly mist the area where the material is to be removed. Allow time for the water to soak into the material. This may take several passes with the hose of the sprayer.

- Using plastic scrapers, kitchen utensils or windshield scrapers are good for this type of work. Gently remove small sections of the wet material. A HEPA vacuum should be running in the immediate vicinity of the work to remove airborne fibers which may be released during the operation.

- Immediately place wet asbestos waste into a 6 mil pre-labeled asbestos disposal bag. Vacuum collapse the bag, twist the neck of the bag, tape with duct tape, fold the twisted portion over onto itself and tape again.

- Wipe the outside of the bag with clean damp cloths and place the bag into a second pre-labeled disposable bag. Tape shut the second bag.

- Using nylon brushes and wet rags to clean any residual ACM and PACM from the area. Lightly mist with a bridging encapsulate the area where the material was removed and 6 to 12 inches around the area. This will lock down any fibers, which may have settled, onto the vicinity adjacent to the work area. The HEPA vacuum should be continuously running during the final cleaning and encapsulation work.

- Remove loose material hanging from the friable ACM and PACM with the suction from the HEPA vacuum.

5.5 After work is completed:

- HEPA vacuum, head, arm and shoulders before leaving area under containment.

- HEPA vacuum ladder while climbing down.

- While standing in plastic sheet, thoroughly HEPA vacuum any ladder and pass it to person standing out side of protected area.

- Remove the protective clothing, gloves, etc. and follow "Dry Decontamination Procedures.

- Mist the plastic sheeting, carefully roll up the sheeting with the rags or cloth used.

- Remove barricades, tape, signs, and replace items that were moved out of the way.

The building management should consider using asbestos O&M work control forms similar to those which may already be in use for non-ACM work in the building, or expanding existing forms to include the content of the request and approval forms illustrated in Appendices A1-A5.

6.0 AIR MONITORING

Air monitoring is done for the purposes of determining if airborne asbestos fibers exceed the OSHA action level of Permissible Exposure Level (PEL). Class I and II type disturbances of ACM are not to be accomplished by Essex workers. Any projects that may cause ACM disturbances beyond OSHA action levels will be reported to the Program Manager and turned over to BENCHMARK.

7.0 MEDICAL SURVEILLANCE

The requirements for medical surveillance are based upon exposure of workers to airborne fiber contents at the OSHA action level. Class I and II type disturbances of ACM are not to be accomplished by Building Owner workers. Any projects that may cause ACM  disturbances beyond OSHA action levels will be reported to the Asbestos Program Manager and turned over to BENCHMARK.

8.0 WORKER PROTECTION

This section covers the personal protection equipment including respirator use and subsequent respirator program. It is the policy of the Essex Property Trust Management to prevent their employees, subcontractors, or tenants to be exposed to ACM beyond the action level and/or PEL. Therefore, the need for respirator program requirements is not applicable at this time.

9.0 EXPOSURE MONITORING

When an activity or work requirement is beyond the small scale asbestos procedure (see Section 5.1 of this action plan) and may exceed the OSHA action level and/or PEL of 0.1 f/cc, the management individual trained as a Class III competent person will notify the Program Manager. The Program Manager will notify BENCHMARK who will set up a negative exposure assessment to establish if exposure is probable.

10.0 SPECIAL CLEANING PROCEDURES

10. 1 VACUUMING ACM and/or PACM

Where vacuuming methods are necessary, HEPA filtered vacuuming equipment must be used an1d emptied in a manner that minimizes the reentry of asbestos into the workplace.

10.1.1 CHANGING FILTERS ON HEPA-VACUUMS

Change filters of HEPA-vacuums on a 6-mil polyethylene drop sheet using the following procedures:

1. Select an out of the way area in a routine maintenance area,

2. Wet the filter with a fine mist of water,

3. Remove filter from vacuum and place immediately into a properly labeled 6-mil polyethylene bag, and

4. Seal the bag and place within a second properly labeled bag.

10.2 CARE OF ASBESTOS-CONTAINING FLOORING MATERIALS

All vinyl and asphalt flooring shall be maintained as ACM or Suspect ACM unless proper sampling protocol demonstrates the absence of asbestos. The following procedures must be followed during cleaning:

- Sanding of flooring material is prohibited,

- Stripping of finishes shall be conducted using low abrasion pads at speeds lower than 300 rpm and wet methods,

- Burnishing or dry buffing may be performed only on flooring that has sufficient finish so that the pad cannot contact the flooring material.

10.3 DISPOSAL OF ASBESTOS WASTE

Once the asbestos-containing waste material has been removed from the areas of concern, there are certain precautions that must be taken before disposal operating begins. The first and probably most important, is to ensure that all of the waste material has been thoroughly treated with water. By ensuring this, the chances of airborne asbestos fiber generation are significantly reduced. The waste material is now suitable for containerizing.

The following are important points to remember in bagging asbestos waste for disposal.

- Asbestos containing waste must be handled in a careful manner to keep airborne fiber generation minimal.

- Some materials should not be included in the bags, (i,e., metal, sharp objects, etc )

- Waste-containing bags should be sealed with duct tape. All excess air is squeezed out of bags before they are sealed. Use HEPA vacuum system to collapse the bags.

- Make sure the asbestos warning label on each bag is legible, so that no bags will be disposed of mistakenly.

- Hose the bag down, wet wipe or HEPA vacuum clean.

- Store bags in designated location for removal to a qualified land fill.

The Environmental Protection agency defines friable asbestos as "material containing more than 1% percent asbestos by weight that hand pressure can crumble, pulverize, or reduce to powder when dry. For purposes of defining asbestos containing waste that must be treated as "hazardous" material, the Department of Health services uses the following definition: material that is classified as friable finely divided and powdered waste that contain greater than one percent (1.0%) asbestos.

Asbestos waste weighing less than 50 pounds does not need to be transported by a registered hazardous waste hauler. However, any waste, which is transported to a disposal site licensed to receive hazardous waste, must be accompanied by a properly completed Uniform Hazardous Waste Manifest. These Manifests are available from the Department of Health Services, (916) 739-3145.

Whenever the asbestos waste exceeds one percent asbestos by volume, the "waste generator" is required to report the amounts being removed to the State Board of Equalization.

Asbestos waste weighing less than 50 pounds does not need to be transported by a registered hazardous waste hauler. However, any waste, which is transported to a disposal site licensed to receive hazardous waste, must be accompanied by a properly completed Uniform Hazardous Waste Manifest. These Manifests are available from the Department of Health Services, (916) 739-3145.

Whenever the asbestos waste exceeds one percent asbestos by volume, the "waste generator" is required to report the amounts being removed to the State Board of Equalization.

APPROVED ASBESTOS WASTE HAULERS

Eight Ball Line Trucking

2717 Goodrick Avenue

Richmond, CA 94804

(510) 528-2483

Pacific Rim Environmental Services

3690 Sprig Drive

Benecia, CA 64510

(707) 746-5808

National Environmental*

P.O Box 4545

Redding, CA 96099

(916) 222-6613

* Large quantity hauler

10.4 ACCIDENTAL ASBESTOS RELEASE RESPONSE ACTIONS.

The following emergency plan of action is to be followed in the case of an accidental release of asbestos-containing fiber into the air. The plan of action is designed to allow trained and aware personnel to contact the proper individuals and to perform proper tasks with little or no time lost between discovery of the incident and remediation

1) Isolate the area to the extent feasible in an orderly and calm fashion.

2) Contact the Asbestos Program Manager immediately.

3) The Asbestos Program Manager will determine if BENCHMARK will need to be contacted.

4) The Asbestos Program Manager will assign trained maintenance personal to isolate the area per procedures and will determine if the incident is major or minor.

5) If the incident is assessed as a "Major Incident", BENCHMARK will dispatch a project manager who will respond in person to assess the situation and make recommendations for further action.

6) If the incident is assessed as a "Minor Incident", the Asbestos Program manager will assign trained maintenance personnel to perform cleanup procedures.

7) Clearance air monitoring will be performed by BENCHMARK.

10.5 ACCIDENTAL ASBESTOS RELEASE PROCEDURES

The following procedures are use in performing an Emergency Cleanup Task.

When asbestos debris is found on the floor or on any surface, the Asbestos Program Manager should be immediately notified. The Asbestos Program Manager will assign a trained maintenance person to isolate the area immediately.

A" Minor Incident" is one that can be effectively dealt with using in-house Class III trained personal. A small gouge in a section of exposed pipe insulation, a small amount of blown on ceiling material less than (one square foot) or removal of less than three square feet of floor tile are examples of minor incidents.

A "Major Incident" is one which cannot be effectively dealt with by Class III trained personal. Removal of pipe insulation in quantities greater than three lineal feet or the clean up of more than one square foot asbestos debris.

Once the incident has been determined a "Minor incident" proper clean up of the material should be performed by only Class III maintenance personnel.

11.0 ASBESTOS DEBRIS FOUND should be cleaned in the following manner.

Perform all work of decontaminating objects ever possible, on a 6-mil drop sheet.

- Pick up the object, if possible and HEPA vacuum all surfaces. If not HEPA vacuum in place.

- Hand object to worker who will wet debris and place in to 6-mil bag.

- D Isolate and restrict entry into the area and post warning signs to prevent access into the area by persons other than those necessary to perform the response action.

- Shut-off or temporarily modify the ventilation system to prevent the flow of air into or out of the area.

The response action for any major fiber release episode should be designed by an asbestos consultant and conducted by an asbestos abatement contractor.

12.0 HYGIENE PRACTICES

Hygiene practices for this O&M Plan prohibit smoking in areas where workers may be occupationally exposed to asbestos due to activities in the area.

Additional hygiene practices are mandatory for all other Classes of work (not prescribed in this O&M Plan) and must be implemented for maintenance workers if they are to perform other work than limited Class III and Class IV operations.

13.0 LARGE-SCALE DISTURBANCES AND UNSPECIFIED OPERATIONS

13.1 LARGE-SCALE DISTURBANCE OF ACBM and PACM

Maintenance and renovation activities which do not meet all the criteria for small-scale, short-duration activities should be considered large-scale disturbances. An asbestos consultant should always be used when developing plans for abatement projects other than small-scale short duration.

The planning of an asbestos removal project is critical to completing the project safely and cost-effectively. A written asbestos removal plan should be prepared that describes the equipment and procedures that will be used throughout the project. The asbestos abatement plan will aid not only in executing the project but also in complying with the notice and reporting requirements of the USEPA asbestos regulations (40 CFR 61, Subpart M) and local air districts, which call for specific information such as a description of control methods and control equipment to be used and the disposal sites the contractor proposes to use to dispose of the asbestos containing materials.

13.2 UNSPECIFIED OPERATIONS

Operations which meet all the criteria for small-scale, short-duration activities but which are not of similar nature to activities described in this O&M Plan should be considered unspecified operations.

The Asbestos Program Manager should modify this O&M Plan to include specific written procedures for the unspecified operation or engage an asbestos consultant to do so.

14.0 REPLACEMENT MATERIALS

In 1989, EPA promulgated the Asbestos Ban and Phaseout Rule (40 CFR 763.160 to 763.179). This rule applies to new product manufacture, importation and processing, and essentially bans almost all asbestos-containing products in the United States by 1997. The EPA also prohibits the installation or reinstallation of asbestos-containing insulation and the spray application of asbestos-containing materials to buildings, structures, pipes and conduits (40 CFR 61.146 and 61.148). These rules do not require the removal of ACM currently in place in buildings.

The building management should ensure and document that no ACM is installed in the building subsequent to the initiation of this O&M Program. Records should be kept on each new or replacement material installed in the building including the manufacturer's and contractor's statement that the material is asbestos free and the locations where the material is installed. If possible, new and replacement materials should differ in physical appearance from ACM to facilitate future differentiation.

15.0 RECORDKEEPING

The Asbestos Program Manager should maintain records described in this section indefinitely if not specified otherwise. The Asbestos Program Manager may utilize the services of competent organizations such as industry trade associations and employee associations to maintain the records required by this section.

The building management should comply with the requirements concerning transfer of records set forth in 29 CFR 1910.20(h). Whenever the building management ceases to do business and there is no successor to receive and retain the records for the prescribed period, the building management should notify the Director of the National Institute for Occupational Safety and Health (NIOSH) at least 90 days prior to disposal and, upon request, transmit them to the Director.

The Asbestos Project Manager should maintain a record of each proposed renovation or maintenance project including:

1. A copy of the Request Form,

2. Any additional records required in this section if asbestos is disturbed.

15.1 PROGRAM PARTICIPANTS

The Asbestos Program Manager should maintain a record of each participant in the O&M Program including:

1. Name,

2. Position, and

3. Dates of Service

Notification Program:

The Asbestos Program Manager should maintain a record of steps taken under the Notification Program including:

1. Copies of any notices set to building occupants,

2. Dates and attendees of any informational seminars, and

3 Dates and general locations of any warning labels applied.

15.2 VISUAL MONITORING

The Asbestos Program Manager should maintain a record of each surveillance including:

1. Documentation of any changes in the condition of ACM,

2. Notation of any response actions undertaken since the initial inspection or the most recent periodic surveillance,

3. Date of the surveillance, and

4. Name of individual conducting the surveillance.

The Asbestos Program Manager should maintain a record of each re-inspection including:

1.  Location and type of ACBM AND PACM remaining in the building,

2.  A hazard assessment of ACBM AND PACM remaining in the building,

3.  Recommendations and timetable for response actions which should be scheduled within the next three years,

4.  Date of the re-inspection, and

5.  Name of individuals conducting the re-inspection.

15.3 WORKER TRAINING

The Asbestos Program Manager should maintain a record of each individual who receives training including:

- Individuals name and job title,

- Completion date and location of training,

- Type of training and training provider,

- Date of the analysis,

- Signature of the analyst,

- An evaluation of the results.

This O&M Plan was revised for the purpose of in-place management of all ACBMs, ACMs and PACMs in buildings present on the Building Owner's property. The limited sampling performed should not be construed as representation of all non-asbestos containing materials located throughout buildings.

APPENDIX A-1 A sample application form for vendors and contractors maintenance work approval.

--------------------------------------------------------------------------------

Job Request Form for Maintenance Work

Name: ____________________________      Date:___________________

Telephone No.:

Name and Telephone No. of Supervisor:

Requested Starting Date:______      Anticipated Finish Date:_______

Description of area where work is to be performed:

Description of Work:

Description of asbestos-containing material present, if known (location and type):

Submit this application to:

Asbestos Program Manager

An application must be submitted for all vendor/contractor work where asbestos-containing material is present and whether or not it may be disturbed. An authorization must then be received before any work can proceed.

Do Not Write Below This Line

__________________________________________________________________________________________

Job Request No.

Granted  / Denied

APPENDIX A-2 - A sample release form for outside tradesmen and vendors

--------------------------------------------------------------------------------

Asbestos Release Form

Name:

Company:

Address:

Scope of Services:

Asbestos-containing and/or presumed asbestos-containing material is present in the area where work is to be performed. Asbestos-containing material in good condition which is left undisturbed presents minimal exposure hazard. When asbestos-containing material is disturbed or damaged, asbestos fibers may be released into the air creating a health hazard.

Inhaling asbestos fibers has been linked with various types of cancer. If you smoke and inhale asbestos fibers, the chance that you will develop lung cancer is greater than that of the non-smoking public. The following attached type(s) of asbestos-containing material are present in the area where work is to be performed.

By signing section 1 of this form, I hereby verify that I have been made aware of the presence and hazard of asbestos-containing material and presumed asbestos-containing materials in the area where work is to be performed; also the type(s) of asbestos-containing material (ACM) and presumed asbestos-containing material present. I will avoid damaging the ACM/suspect ACM in any way. If I do damage the ACM in any way, I will immediately report such occurrence to the Assistant Asbestos Program Manager of the complex.

Section 1.

Name:

Title:

Signature:

Date.

By signing Section 2 of this form, I am further stating that I have agreed to the following:

1. I have informed my employees of the potential risk and health effects of asbestos,

2. I have informed my employees of the locations and types of asbestos-containing and presumed asbestos containing materials located at the site(s).

Section 2.

Signature:

Date:

APPENDIX A-3 A sample form for recording training information.

--------------------------------------------------------------------------------

Training Record

Name:

Title:

Training type:

Hours:_________  Dates: _______________________

Location Provider:

Attach if a certificate was issued

APPENDIX A-4 A sample acknowledgement for VENDORS AND CONTRACTORS whose duties may involve the disturbance of asbestos-containing material.

--------------------------------------------------------------------------------

Asbestos Acknowledgement Form

Working with asbestos may be dangerous. Inhaling asbestos fibers has been linked with various types of cancer. If you smoke and inhale asbestos fibers, the chance that you will develop lung cancer is greater than that of the non-smoking public.

Federal law requires that: You be supplied with the proper respirator and be trained in its use; You be trained in safe work practices and in the use of equipment used on the job site; You receive a medical examination; These items are to have been done at no cost to you. By signing this certification, you are stating that your employer has met these obligations to you.

RESPIRATORY PROTECTION. I have been trained in the proper use of respirators, and informed of the type of respirator to be used in given conditions. I have a copy of the written respiratory protection manual issued by my employer. I have been equipped at no cost with the respirator assigned to me.

TRAINING COURSE. I have been trained in the dangers inherent in handling asbestos and breathing asbestos dust, and in proper work procedures and personal and area protection measures. The topics covered in the course included the following:

Physical characteristics of asbestos

Health hazards associated with asbestos

Respiratory protection

Use of protective equipment

Negative air systems

Work practices, including hands-on or on-job training

Personal decontamination procedures

Air monitoring, personal and area

MEDICAL EXAMINATION. I have had a medical examination within the last 12 months which was paid for by my employer. This examination included: health history, pulmonary function tests, and may have included an evaluation of a chest X-ray.

Signature:

Printed Name:

SS Number:

Witness:

APPENDIX A-5 A sample acknowledgement for NEW EMPLOYEES

--------------------------------------------------------------------------------

Asbestos Acknowledgement Form

Working with asbestos may be dangerous. Inhaling asbestos fibers has been linked with various types of cancer. If you smoke and inhale asbestos fibers, the chance that you will develop lung cancer is greater than that of the non-smoking public.

TRAINING COURSE. I have been trained in the dangers inherent in handling asbestos and breathing asbestos dust, and in proper work procedures and personal and area protection measures. The topics covered in the course included the following:

Physical characteristics of asbestos

Health hazards associated with asbestos

Types of Asbestos Containing

Materials (ACM)

Identifying damaged ACM

O&M Plan (asbestos related procedures)

Emergency response contacts

Signature Printed Name SS Number:

Witness: