CARPET CLEANING

SCOPE OF WORK:

Clean exposed carpeted surfaces using truck mounted equipment (where possible) and the hot water extraction method.  The equipment and cleaning method shall be capable of removing all water to allow for drying of the carpet within 8 hours of completion of the job.  The contractor shall not use the Bonnet Rotary cleaning method.

LOCATION OF WORK:

To be determined by the owner on an as needed basis

EXTRACTION EQUIPMENT SPECIFICATIONS:

Type of unit			Truck mount where possible

Water pressure			250psi minimum, 450 recommended

Suction pressure		150psi minimum, 200 recommended

Air movement			100 CFM minimum

Water flow			1 GPM minimum

Solution tank			12-gallon minimum

Recovery tank			12-gallon minimum

Wand/Floor tool 		Scrub Wand

Heat	Capable of achieving continuous cleaning temperatures at the face of the carpet of 130 degrees minimum

OTHER VENDOR SPECIFICATIONS:

IICRC Certification (Institute of Inspection, Cleaning and Restoration Certification)

STATEMENT OF WORK:

Work includes but is not limited to the cleaning of:

- Surface preparation

- Pre-spotting stains

- Agitation

- Extraction

- Grooming

Clean or cleaning as used herein means to remove all foreign matter and to remove signs of a previous resident.

DESCRIPTION OF WORK:

# A.  Surface Preparation:

- Remove all removable items in areas to be cleaned.

- Thoroughly inspect the carpet surface to ensure that it has been vacuumed properly, and if not, vacuum carpet using a beater/bar upright to remove all loose dirt and debris.

- Vacuums should be approved in the Carpet and Rug Institutes' Green Label Program.

- Call CRI's toll free number 1-800-882-8846 to verify approval of the vacuum cleaner model using the I.D. number.

- A listing of machines in compliance with the program standard is also available at: www.carpet-rug.com

- Vacuum bags should be replaced when 1/2-2/3 full.

- Protect all adjacent surfaces to prevent damage to newly painted surfaces or adjacent flooring materials.

B.  Application of pre-spotting Mix:

1.  Prepare pre-spotting Mix as specified by manufacturer:

- Solution Ph level less than 10 (preferably close to 9) and leaves a minimum of residue

- Do not use extra pre-spotter or traffic lane cleaner.

- Do not use a detergent that dries sticky

- Do not use any cleaning agents that contain optical brighteners

- Spray the soiled areas using a pump up sprayer.

- Pre-treat extremely heavy stains using a 32 oz. trigger spray bottle filled with 12 oz. degreaser and 4 oz. pre-spotter mixed with 16 oz. hot water.  Allow the solution to sit 4 to 5 minutes then rinse thoroughly before proceeding with the standard pre-spotting.

C.  Agitation and Dwell:

- Traffic lane cleaner must be raked into the carpet’s pile.

- Agitate spots thoroughly.

- Allow to sit (or dwell) for 10-15 minutes

- If pre-spray dries before extraction, spray area with clean water (not more pre-spray)

D.  Extraction:

- Use water only or a low pH rinse in the extraction machine.

- Water temperature should be 130-140 degrees at the face of the carpet.

- The machine should deliver a minimum of 1 gallon of water per minute.

- Take multiple strokes to remove the embedded dirt and the traffic lane cleaner.  Pay special attention to the dirtiest areas.

- Take extra vacuum passes to remove as much moisture as possible.

- Reduce the drying time by using air movers if possible.

- No silicone based anti-soil treatments should be used on Essex carpets.

E.  Grooming:

1.  On cut-pile carpets, rake the carpet to remove signs of foot traffic.

2.  Use shoe protectors after the carpet is cleaned.

## APPENDIX H – UNIT PRICE SHEET

Carpet Cleaning:

- 1 bedroom/1 bath - $______________

- 2 bedroom/1 bath - $______________

- 2 bedroom/2 bath - $______________

- 3 bedroom/2bath - $_____________

- Set dye spot removal - $______________

Carpet Dyeing:

- 1 bedroom/1 bath - $______________

- 2 bedroom/1 bath - $______________

- 2 bedroom/2 bath - $______________

Pet Odor Removal:

- Pet Odor Deodorize

- 1 bedroom/1 bath - $______________

- 2 bedroom/1 bath - $______________

- 2 bedroom/2 bath - $______________

- Surface Pet Treat  - 2 day process

- 1 bedroom/1 bath - $______________

- 2 bedroom/1 bath - $______________

- 2 bedroom/2 bath - $______________

Other Services:

- Patching  $______________

- Stretching $______________

- Power Stretches $______________

- Molding Replacement $______________

- Seams $______________

- Metal Strips $______________

- Reducer $______________

- Spot Dyeing $______________

- Kool-aid Removal $______________

- Bleach Stains $______________

- Pad Replacements $______________

- Other fixed charges not included in standard rate - $______________

- Other hourly charges not included in standard rate - $______________

Overtime Emergency Water Extraction:

- Hourly Rate - $_________________

- Applies before 7 a.m. and after 5 p.m. Mon.-Fri., and Saturdays, Sundays and Holidays.