Termite re-treatment of buildings (Termidor ®)

Provide proof of current license in the State of ________________

and further proof that license has been in effect for 5 years.

- The chemical to be used for termite treatment shall be in accordance to all national, state, and local code requirements and approved by the EPA.

- To the extent approved by governmental agencies having jurisdiction, use  working solutions containing the following chemical:

Termidor ® SC @ 0.06% -  0.125% solution or

Termidor ® 80WG @ 0.06% - 0.125% solution

Per manufacturer’s recommendations for specific application

3.	Termidor ® shall be applied by technicians trained  in the methods of trenching, rodding, short rodding, sub-slab injection, low pressure banded surface applications and foam delivery techniques in a manner that will provide a continuous chemical barrier consistent with the manufacturer’s recommendations.

Installation of the product:

- Remove all wood and cellulose containing materials, including scrap wood and form boards from around the slab area or beneath the crawlspace area.  Notify the Project Manager of any wood that is in contact with soil that is attached to the building.

- Do not treat soils that are frozen or water saturated.  All clay soils should be checked for water saturation by digging a hole 24 inches in depth and allowing it to stand overnight.  If water is found in the hole the next day the treatment method should either be postponed or the volume should be adjusted to assure a proper treatment.

- Slab on Grade Construction applications shall be made using techniques such as sub-slab injection, rodding and/or trenching.  Injectors shall not extend beyond the tops of the footings.

- Treat along the outside of the foundation to form a continuous termiticide barrier in the soil.

- For shallow foundations, 1 foot or less, dig a narrow trench approximately 6 inches dep or to the bottom of the footing along the outside of the foundation walls.  Do not dig below the bottom of the footings.  The dilution shall be applied to the trench and mixed with the soil as it is replaced in the trench.

- For foundations with footings deeper than 1 foot, apply the dilution at a rate of 4 gallons per 10 linear feet per foot of depth.  The applicator must trench and rod into the trench or along the foundation walls and around columns at the rate prescribed from grade to the top of the footing.  If the footing is more than 4 feet in depth, then the treatment shall extend to a depth of 4 feet below grade.  Rod holes shall have a maximum spacing of 12 inches.

- Sub – slab injection can be made from inside the structure or outside as required.  The injections can be made using either vertical drilling and injection from the interior along the perimeter walls or by horizontal drilling, rodding and injection from the exterior of the foundation and building.

- (Alternate bid price) All Bath traps should  be treated by application of the solution at a rate of 4 gallons of dilution per 1 square foot of exposed surface area.  All wood blocking should be removed prior to the treatment.

- When treating cracks and expansion joints in the slab, along sidewalks or patios adjacent to the exterior foundation wall or other areas where holes are to be drilled to form a continuous termiticide barrier, the holes should be spaced at intervals up to 12 inches depending on the soil type.

- Hollow block foundations shall be treated in the void directly above the

- footing.  Drill and treat all voids in multiple masonry elements of the

- structure extending from the structure to the soil in order to create a

- continuous barrier in the area to be treated.  Apply at a rate of 2 gallons

- per 10 linear feet of footing using a nozzle pressure of 25 psi.  All access

- holes shall be drilled below the sill plate as close as is feasible to the top of

- the footing.  Do not treat voids insulated with rigid foam in this manner.

- Basement walls shall be treated at a rate of 4 gallons of dilution per 10

- lineal feet per foot of depth.  The application shall be made by trenching or

- rodding at a rate of 4 gallons of dilution per 10 linear feet per foot of depth

- to a maximum depth of 4 feet.

- Crawl Spaces shall have a vertical termiticide barrier applied at the rate of

4 gallons of emulsion per 10 linear feet per foot of depth from grade to the

top of the footing, or if the footing is more than 4 feet below grade, to a

minimum depth of 4 feet.

- Apply emulsion by trenching and rodding into the trench along both

- sides of the foundation and around all columns.

- Rod holes and trenches shall not extend below the bottom of the

- footing.

- Rod holes shall be spaced so as to achieve a continuous chemical

- barrier but in no case more than 12 inches on center.

- Trenches shall be a minimum of 6 inches in depth and excavated to

- provide a level area in which to apply the termiticide.  The trench shall

- be stepped as required to assure that the termiticide does not run off

- on a slope.  The emulsion shall be blended with the soil upon

- backfilling of the trench.

- Drill all sidewalks and patio slabs at a spacing of 12 inches on

center, depending upon soil type and rodding the soil to assure a

thorough and continuous coverage of the area to be treated.

- Inaccessible crawl space areas shall be treated to create a continous

horizontal barrier on the soil surface.  The barrier can be placed using

a spray application with pressures less than 25 p.s.i. per the

manufacturer’s recommendations.

- Utility Entrances

- Apply 4 gallons of dilution per entry point to assure adequate coverage in these areas.  The applicator shall locate  all plumbing and utility entrances prior to commencing the application of the termiticide to assure that these areas are treated separately as prescribed above.

- Miscellaneous: Apply toxicants at the recommended rate at the following areas:

- Immediately below or adjacent to all expansion joints, control joints, or cracks, drill and apply the dilution to assure proper coverage in these areas.

- Where exterior facings or veneers extend below grade level along the exterior side of all foundation walls the cavity shall be treated by drilling holes along the wall at an angle allowing penetration of the cavity space.

- Upon completion of the work, a written guarantee for a period of 5 years to the Owner shall be delivered to the Owner providing:

- Retreatment if termite infestation occurs during the guarantee period at no additional cost to Owner.

- Annual re-inspection for evidence of termites during the Guarantee Period.