Removing and replacing a toilet

- Tools You Need to Remove and Install a Toilet

- Putty knife

- Wrench for toilet bolts

- Wax bowl ring

- Flange bolts

- Flexible water supply tube

- Plastic toilet shims

- How to Remove and Install a Toilet: Step by Step

- Turn off the water supply to the toilet you plan to replace or remove for flooring installation.

- Unhook the flexible supply line connected to the toilet tank.

- Flush the toilet to drain the tank.

- Remove excess water from the toilet bowl (a turkey baster works great)

- Remove the toilet bowl by removing the bolts holding the toilet to the floor flange. These bolts may need to be cut to remove the nuts due to corrosion.

- Plug the drainpipe with an old rag so that sewer gas doesn’t leak in.

- Remove the used toilet flange bolts. Then use a putty knife to scrape up the remains of the old wax gasket from the floor flange. The wax is usually yellow or off-white and should clean up easily. The area does not have to be spotless before installing the toilet, but it should be relatively clean.

- Check the area for water damage and make any necessary repairs to the floor. Water damage from a leaky toilet can weaken the floor and facilitate mold growth. It is important to address these issues before installing new flooring or replacing the toilet. Inspect the floor flange and bolts that anchor the flange to the floor. If the flange or bolts are corroded to a point where they can no longer be used to obtain a seal, they will need to be replaced.

- If the flange is found to need replaced, there are two options available. One is to replace the entire flange by cutting off the old flange and gluing a new one in its place. This may require you to seek a professional plumber, depending on your individual situation. The other option is, if the flange has damage to one side or minimal damage, but the bolts will no longer stay attached to the flange; HD Supply carries a flange repair kit, HDS part # 569360, which may be used for this type repair.

- Take the new wax bowl ring out of the package and inspect it. If the wax ring has   a plastic collar, which is the recommended replacement, check to be sure that it  fits over the floor flange. Install the new wax ring onto the toilet flange.

- Turn the toilet bowl right side up and lower it on top of the flange, using the bolts on the floor flange as guides. The bolts should come with pieces of plastic that help them remain upright while the toilet is being installed. If the bolts move during the installation, have a helper hold them steady. This may take a few attempts.

- Once the toilet bowl is in position, press down firmly and rock it back and forth a little bit to help shimmy it into place and secure it.  Use the shims to level it if necessary due to an unleveled floor.  Be sure to cut the shims so they fit under the toilet and aren’t visible.

- Place the washers and then the nuts on the bolts protruding from the toilet and hand-tighten them. Turn the bolts once or twice with a wrench. Make certain the bolts are tight enough to prevent the toilet from rocking, but do not over-tighten them or the bowl can crack. These bolts may need to be cut off to after the bolts are tightened to allow for the plastic caps to be installed.

- Apply a dab of plumber’s putty around the base of the decorative caps for the bolts and place them over the bolts. Press the caps firmly in place.

- It is a good idea to replace the flexible water supply line at this time with a new one. Attach the flexible supply tube to the water supply (the pipe coming out of the floor or wall that brings water to the toilet) and tighten the coupling nut on the end. If the nut is loose, the connection will leak.

- Attach the other end of the flexible supply tube to the tank. If you are facing the tank, the water supply tube attaches to the left on the underside of the tank.

- Turn on the water supply to the toilet and check for leaks.

- Flush the toilet a few times and check for leaks at the base of the toilet

- Caulking the base of the toilet to the new flooring is discouraged; this could hide a leaky wax ring from detection and lead to the flooring being rotted below the toilet before detection.